I2C Controller

| Revision | Date | Owner | Author | Change Description | Project |
| --- | --- | --- | --- | --- | --- |
| 0.1 | July 10 2024 | Cheng Qian | Shuwang Qin | New structure, add DMA |  |
| 0.2 | July 31 2024 | Cheng Qian | Shuwang Qin | Add hw_trig | T6W |
| 0.3 | Oct 16 2024 | Cheng Qian | Shuwang Qin | Add APB sequential mode to set FIFO |  |
| 0.4 | Oct 31 2024 | Cheng Qian | Shuwang Qin | Add tee lock |  |
| 0.5 | Nov 7 2024 | Cheng Qian | Shuwang Qin | Change tee lock to tee open | GXLX4 |
| 0.6 | Dec 26 2024 | Cheng Qian | Shuwang Qin | Correct some doc errors |  |
| 0.7 | Mar 5 2025 | Cheng Qian | Shuwang Qin | Fix timing problem | T6X |
| 0.8 | Jun 26 2025 | Cheng Qian | Shuwang Qin | Add task_req_ack; I2C with DMA | X1 |
| 0.9 | Jul 1 2025 | Cheng Qian | Shuwang Qin | Fix 3.4M timing, slave rx phy_done timing; | C4 |
| 1.0 | Jul 22 2025 | Cheng Qian | Shuwang Qin | Fix long delay, add task_done | A9 |

Features

Ports

| Signal | Name | Master mode | Slave mode |
| --- | --- | --- | --- |
| SCL | Serial CLK | I/O | I/O |
| SDA | Serial DATA | I/O | I/O |

Clock

Clocks used in I2C controller

| Clock | Source | Frequency |
| --- | --- | --- |
| clk | Sys_clk | 167MHz |
| SCL | Clk/b_ratio | 40K - 20.875MHz |

Tx and Rx

I2C PHY TX path

| Signals | Width | Source |
| --- | --- | --- |
| Tx_data | 8 | Core |
| Tx_data_vld | 1 | Core |
| Tx_data_rdy | 1 | PHY |

I2C PHY RX path

| Signals | Width | Source |
| --- | --- | --- |
| Rx_data | 8 | PHY |
| Rx_data_vld | 1 | PHY |
| Rx_data_rdy | 1 | Core |

FIFO write mode

There are 3 modes for setting FIFO, 2 APB modes and 1 DMA modes.

| Mode | Data source | Operation |
| --- | --- | --- |
| APB 1 : random mode | CPU | b_dma = 0; FIFO address initial: PADDR = BASE + ‘h10, tx_rd_addr = PWDATA PADDR = BASE + ‘h14, tx_wr_addr = PWDATA PADDR = BASE + ‘h18, rx_rd_addr = PWDATA PADDR = BASE + ‘h1C, rx_wr_addr = PWDATA NOTE: must guarantee (tx_wr_addr - tx_rd_addr) >= data_length set PADDR[7] = 1, PADDR[6:0] = fifo_adr; data_fifo[fifo_adr + 0] = PWDATA[8*0 +: 8] data_fifo[fifo_adr + 1] = PWDATA[8*1 +: 8] data_fifo[fifo_adr + 2] = PWDATA[8*2 +: 8] data_fifo[fifo_adr + 3] = PWDATA[8*3 +: 8] |
| APB 2 : sequential mode | CPU | b_dma=0; FIFO address initial; Wr( BASE+’h20, data), tx_wr_addr increase 1 |
| DMA | DDR | b_dma = 1; FIFO address initial |

FIFO read mode

There are 3 modes for setting FIFO, 2 APB modes and 1 DMA modes.

| Mode | Data source | Operation |
| --- | --- | --- |
| APB 1 : random mode | CPU | b_dma = 0; FIFO address initial: PADDR = BASE + ‘h10, tx_rd_addr = PWDATA PADDR = BASE + ‘h14, tx_wr_addr = PWDATA PADDR = BASE + ‘h18, rx_rd_addr = PWDATA PADDR = BASE + ‘h1C, rx_wr_addr = PWDATA set PADDR[7] = 1, PADDR[6:0] = fifo_adr; PRDATA[8*0 +: 8] = data_fifo[fifo_adr + 0] PRDATA[8*1 +: 8] = data_fifo[fifo_adr + 1] PRDATA[8*2 +: 8] = data_fifo[fifo_adr + 2] PRDATA[8*3 +: 8] = data_fifo[fifo_adr + 3] |
| APB 2 : sequential mode | CPU | b_dma=0; FIFO address initial: PADDR = BASE + ‘h10, tx_rd_addr = PWDATA PADDR = BASE + ‘h14, tx_wr_addr = PWDATA PADDR = BASE + ‘h18, rx_rd_addr = PWDATA PADDR = BASE + ‘h1C, rx_wr_addr = PWDATA Rd( BASE+’h24, data), rx_rd_addr increase 1 |
| DMA | DDR | b_dma = 1; FIFO address initial: PADDR = BASE + ‘h10, tx_rd_addr = PWDATA PADDR = BASE + ‘h14, tx_wr_addr = PWDATA PADDR = BASE + ‘h18, rx_rd_addr = PWDATA PADDR = BASE + ‘h1C, rx_wr_addr = PWDATA |

Register Map

| Address | Name | Note |
| --- | --- | --- |
| BASE + 0x0 | RDY | Ready and tee |
| BASE + 0x4 | I2C | Set DMA and join |
| BASE + 0x8 | Start | Write at last |
| BASE + 0xC | Bus | Set slave mode |
| BASE + 0x10 | tx_rd_addr | tx fifo pull address |
| BASE + 0x14 | tx_wr_addr | tx fifo push address |
| BASE + 0x18 | rx_rd_addr | rx fifo pull address |
| BASE + 0x1C | rx_wr_addr | rx fifo push address |
| BASE + 0x20 | tx | empty, full, tx_data |
| BASE + 0x24 | rx | empty, full, wch_data |
| BASE + 0x30 | Irq_sts | Write 1 to clear irq |
| BASE + 0x34 | Irq_enable | Irq mask |
| BASE + 0x38 | shake | RO |
| BASE + 0b1xxxxxxx | FIFO addr | Set fifo use APB mode |

CFG_RDY

BASE + 0

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 1 | Tee_open | If 1, tee open, everyone can use; If 0, tee_lock, just PUSER[9:2] = ‘b11 can use. Default 0. | New feature in r0.4 |
| 0 | I2c_rdy | if 0, controller own i2c | Not mandatory, for sw use |

CFG_I2C

BASE + 4

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 23:16 | rx_thresh | rx_fifo_irq = rx_count >= rx_thresh | for software, verificaition don’t care |
| 15:8 | tx_thresh | tx_fifo_irq = tx_count <= tx_thresh | for software, verificaition don’t care |
| 7:5 | RESV |  |  |
| 4 | b_dma | set 1 to use DMA | only in DMA mode |
| 3 | b_hw_neg | use hw_trig’s negedge to start | only in hw_trig mode |
| 2 | b_hw_pos | use hw_trig’s posedge to start | only in hw_trig mode |
| 1 | b_join_rx | if 1, the whole fifo is used by RX | strech rx fifo |
| 0 | b_join_tx | if 1, the whole fifo is used by TX | strech tx fifo |

CFG_START

Base + 8

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 31 | b_pending | set 1 to start trans |  |
| 30:24 | RESV |  |  |
| 23:12 | b_length | number of blocks | if 0, the number is 0 |
| 11 | b_10bits | 10 bits slave addr mode | reserved for future update |
| 10:1 | b_slave_addr | 7bits or 10 bits | this version just support 7bits |
| 0 | b_mode | transfer mode | 0: write; 1: read |

CFG_BUS

BASE + 0xC

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 17 | speed_mode | 1:100k | Fix su/hd timing in T6X/C4 |
| 16 | b_slave_mode | 1: I2C slave mode | Default 0: master |
| 15:12 | b_filter_sel | used in din_filter to avoid glitch | dout is delayed (b_filter_sel + 2) sys_clk by din |
| 11:0 | b_ratio | SCL = clk/b_ratio | if b_ratio<=8, SCL = clk/8 |

TX&RX ADDR

BASE +

| ADDR | Name | Details | Note |
| --- | --- | --- | --- |
| 10 | TX_RD_ADDR | tx fifo pull address |  |
| 14 | TX_WR_ADDR | tx fifo push address |  |
| 18 | RX_RD_ADDR | rx fifo pull address |  |
| 1C | RX_WR_ADDR | rx fifo push address |  |

TX_FIFO

BASE + 0x20

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 31:20 | RESV |  |  |
| 19:11 | tx_count |  |  |
| 10 | apb_tx_err | apb_tx_fifo & tx_full |  |
| 9 | tx_empty |  |  |
| 8 | tx_full |  |  |
| 7:0 | tx_data |  |  |

RX_FIFO

BASE + 0x24

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 31:10 | RESV |  |  |
| 19:11 | rx_count |  |  |
| 10 | apb_rx_err | apb_rx_fifo & rx_empty |  |
| 9 | rx_empty |  |  |
| 8 | rx_full |  |  |
| 7:0 | rx_data |  |  |

IRQ_STS / IRQ_ENABLE

BASE + 0x30 / BASE + 0x34

| Bits | Name | Note |
| --- | --- | --- |
| 9 | all_done | task_done & phy_done |
| 8 | task_done | Dma transfer done |
| 7 | phy_done | SPI transfer done |
| 6 | tx_fifo_irq | for software (verification don’t care this feature) |
| 5 | rx_fifo_irq | for software (verification don’t case this feature) |
| 4 | tx_full |  |
| 3 | tx_empty |  |
| 2 | rx_full |  |
| 1 | rx_empty |  |
| 0 | Phy_err | Nack err |

IRQ_STS always updated by hardware.

SHAKE & BLK_CNT

BASE + 0x38

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 16:4 | Blk_cnt | Number of Bytes | RO |
| 3 | tx_data_vld |  |  |
| 2 | tx_data_rdy |  |  |
| 1 | rx_data_vld |  |  |
| 0 | rx_data_rdy |  |  |

ACPU IRQ is only trigger if IRQ_ENABLE bit is set to 1.

FIFO WRITE

BASE + 0b1xxxxxxx

| Bits | Name | Note |
| --- | --- | --- |
| 31:24 | data_fifo[fifo_adr +3] | when fifo_wr == 1, use APB to set FIFO. fifo_wr = PADDR[7]==1, fifo_adr = PADDR[6:0] |
| 23:16 | data_fifo[fifo_adr +2] |  |
| 15:8 | data_fifo[fifo_adr +1] |  |
| 7:0 | data_fifo[fifo_adr +0] |  |

Transfer Controll

- read i2c_rdy(PADDR==0x00) to own I2C.

- set CFG_I2C(PADDR==0x04). If use DMA, set b_dma = 1.

- set CFG_BUS(PADDR==0x0c). If as slave, set b_slave_mode = 1.

- set IRQ_ENABLE(PADDR==0x34), default 0xf.

- set TX&RX FIFO address(PADDR==0x10,14,18,1C).

- initial FIFO mem, 2 modes:

  - APB random mode, set PADDR[7]=1, PADDR[6:0]=fifo_adr.

  - APB sequential mode, Wr(BASE + ‘h20, data).

  - DMA mode, program data in DDR.

- set CFG_START(PADDR==0x08),  write bit[31] = 1, start the data transfer.

- wait irq, finish.

- after done, write i2c_rdy(PADDR==0x00) 1 to release I2C.

RTL Ports

| Port | Details | Source/Target |
| --- | --- | --- |
| Clk_I2C | I2C clock source | sys_clk; I2C_SCL = clk_I2C / ratio |
| APB bus | 32 bits APB slave | Clk81 167MHz |
| SCL_i | SCL input from slave | From PAD to controller |
| SCL_o | SCL output to slave | From controller to PAD |
| SDA_i | SDA input from slave | From PAD to controller |
| SDA_o | SDA output to slave | From controller to PAD |
| I2C_irq | IRQ | Level IRQ to ACPU |

others

| clk | pha | func |
| --- | --- | --- |
| q0 | pha==0 | tx_data |
| q1 | pha==1/8 len | posedge |
| q2 | pha==1/2 len | start/stop sda change |
| q3 | pha==5/8 len | negedge, start/stop scl change |
| end | pha==len-1 | sclk_cnt, tx_blk |
| sclk_neg | q3 + (sel+2)*sys_clk dly | rx_data |