T6X

Peripheral Register Specification

| Date | Author | Change |
| --- | --- | --- |
| 2025/8/27 | liang.zhao | Base on t6x,up module list |

Table of Contents

Table of Contents

Contents

1. Module address list:	8

2. IR_CTRL	9

IRCTRL_MF_IR_DEC0_LDR_ACTIVE	10

IRCTRL_MF_IR_DEC0_LDR_IDLE	10

IRCTRL_MF_IR_DEC0_LDR_REPEAT	10

IRCTRL_MF_IR_DEC0_BIT_0	10

IRCTRL_MF_IR_DEC0_REG0	11

IRCTRL_MF_IR_DEC0_FRAME	11

IRCTRL_MF_IR_DEC0_STATUS	12

IRCTRL_MF_IR_DEC0_REG1	13

IRCTRL_MF_IR_DEC0_REG2	14

IRCTRL_MF_IR_DEC0_DURATN2	17

IRCTRL_MF_IR_DEC0_DURATN3	18

IRCTRL_MF_IR_DEC0_FRAME1	18

IRCTRL_MF_IR_DEC0_STATUS1	18

IRCTRL_MF_IR_DEC0_STATUS2	18

IRCTRL_MF_IR_DEC0_REG3	19

IRCTRL_MF_IR_DEC0_FRAME_RSV0	19

IRCTRL_MF_IR_DEC0_FRAME_RSV1	19

IRCTRL_MF_IR_DEC0_FILTE	19

IRCTRL_MF_IR_DEC0_IRQ_CTL	20

IRCTRL_MF_IR_DEC0_FIFO_CTL	20

IRCTRL_MF_IR_DEC0_WIDTH_NEW	21

IRCTRL_MF_IR_DEC0_REPEAT_DET	21

IRCTRL_IR_DEC_DEMOD0_CNTL0	23

IRCTRL_IR_DEC_DEMOD0_CNTL1	23

IRCTRL_IR_DEC_DEMOD0_IIR_THD	23

IRCTRL_IR_DEC_DEMOD0_THD0	23

IRCTRL_IR_DEC_DEMOD0_THD1	24

IRCTRL_IR_DEC_DEMOD0_SUM_CNT0	24

IRCTRL_IR_DEC_DEMOD0_SUM_CNT1	24

IRCTRL_IR_DEC_DEMOD0_CNT0	24

IRCTRL_IR_DEC_DEMOD0_CNT1	24

IRCTRL_IR_BLASTER_ADDR0	25

IRCTRL_IR_BLASTER_ADDR1	25

IRCTRL_IR_BLASTER_ADDR2	26

IRCTRL_IR_BLASTER_ADDR3	26

3. UART_A/B/	27

UART_A_WFIFO	27

UART_A_RFIFO	27

UART_A_CONTROL	28

UART_A_STATUS	29

UART_A_MISC	30

UART_A_REG5	31

4. I2C_A/B/C	32

Register Map	32

CFG_RDY 	32

CFG_I2C	32

CFG_START	33

CFG_BUS	33

TX&RX ADDR	33

CFG_TX/RX	33

IRQ_STS/IRQ_ENABLE	34

SHAKE & BLK_CNT	34

FIFO WRITE	34

5. PWM_A/B/C/D/E/F/G/H	36

PWM_PWM_A	36

PWM_MISC_REG_A	36

PWM_DELTA_SIGMA_A	36

PWM_TIME_A	37

PWM_A2	37

PWM_BLINK_A	37

PWM_LOCK_A	37

PWM_TEE_ONLY_A	37

6. SPISG_0	38

Base+0x0	38

Base+0x4	38

CFG_START Base+0x8	38

CFG_BUS Base+0xC	39

Base+0x10	40

Base+0x14	40

Base+0x18	40

Base+0x1C	40

Base+0x20	40

Base+0x24	40

Base+0x28	41

Base+0x2c	41

Base+0x30	41

Base+0x34	41

7. MSR_CLK	42

MSR_CLK_DUTY	42

MSR_CLK_REG0	43

MSR_CLK_REG1	43

MSR_CLK_REG2	43

MSR_CLK_REG3	44

MSR_CLK_REG4	44

MSR_CLK_REG5	44

8. SAR_ADC	45

SAR_ADC_REG0	45

SAR_ADC_CHAN_LIST	46

SAR_ADC_AVG_CNTL	46

SAR_ADC_REG3	47

SAR_ADC_DELAY	48

SAR_ADC_LAST_RD	49

SAR_ADC_FIFO_RD	49

SAR_ADC_AUX_SW	50

SAR_ADC_CHAN_10_SW	51

SAR_ADC_DETECT_IDLE_SW	53

SAR_ADC_DELTA_10	54

SAR_ADC_REG11	54

SAR_ADC_REG12	56

SAR_ADC_REG13	56

SAR_ADC_CHNL01	56

SAR_ADC_CHNL23	56

SAR_ADC_CHNL45	56

SAR_ADC_CHNL67	57

SAR_ADC_RDY:	58

9. TEMP_SENSOR(TOP)	59

TS_CFG_REG1	59

TS_CFG_REG2	61

TS_CFG_REG3	62

TS_CFG_REG4	62

TS_CFG_REG5	62

TS_CFG_REG6	62

TS_CFG_REG7	62

TS_STAT0	62

TS_STAT1	64

TS_STAT2	64

TS_STAT3	64

TS_STAT4	64

TS_STAT5	65

TS_STAT6	65

TS_STAT7	65

TS_STAT8	65

TS_STAT9	65

10. debug_i2c	66

I2C_DBG_T_CTRL	66

I2C_DBG_PWD	66

11. spifc	67

PI_FLASH_CMD	67

SPI_FLASH_ADDR	67

SPI_FLASH_CTRL	67

SPI_FLASH_CTRL1	68

SPI_FLASH_STATUS	68

SPI_FLASH_CTRL2	70

SPI_FLASH_CLOCK	72

SPI_FLASH_USER	73

SPI_FLASH_USER1	74

SPI_FLASH_USER2	75

SPI_FLASH_USER3	75

SPI_FLASH_USER4	75

SPI_FLASH_SLAVE	76

SPI_FLASH_SLAVE1	77

SPI_FLASH_SLAVE2	78

SPI_FLASH_SLAVE3	78

SPI_FLASH_C0~7	79

SPI_FLASH_B8~15	79

- Module address list:

| module name | size(kb) | start | end |
| --- | --- | --- | --- |
| i2c_B(ee) | 1 | FE59A400 | FE59A7FF |
| i2c_A(ee) | 1 | FE59A000 | FE59A3FF |
| uart_b(ee) | 8 | FE58E000 | FE58FFFF |
| pwm_h(ee) | 1/2 | FE58C600 | FE58C7FF |
| pwm_g(ee) | 1/2 | FE58C400 | FE58C5FF |
| pwm_f(ee) | 1/2 | FE58C200 | FE58C3FF |
| pwm_e(ee) | 1/2 | FE58C000 | FE58C1FF |
| spisg(for LD) | 8 | FE586000 | FE587FFF |
| sar_adc(ee) | 8 | FE582000 | FE583FFF |
| temp_sensor_top(ee) | 8 | FE580000 | FE581FFF |
| msr_clk(ee) | 8 | FE308000 | FE309FFF |
| spifc(xip, ao) | 8 | FE0B0000 | FE0B1FFF |
| i2c_AO(for mccs,ao) | 8 | FE026000 | FE027FFF |
| cecb(ao) | 8 | FE024000 | FE025FFF |
| IR_TOP(RX, ao) | 8 | FE022000 | FE023FFF |
| uart_a(ao) | 8 | FE020000 | FE021FFF |
| pwm_d(ao) | 1/2 | FE01E600 | FE01E7FF |
| pwm_c(ao) | 1/2 | FE01E400 | FE01E5FF |
| pwm_b(ao) | 1/2 | FE01E200 | FE01E3FF |
| pwm_a(ao) | 1/2 | FE01E000 | FE01E1FF |
| debug_i2c(ao) | 8 | FE016000 | FE017FFF |

- IR_CTRL

1 demodulator and 2 multi-format decode, support 1 path rx input

IRCTRL_MF_IR_DEC0_LDR_ACTIVE

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 28:16 | R/W | 0 | Max duration of Leader’s active part (The value is in unit of TRATE) |
| 12:0 | R/W | 0 | Min duration of Leader’s active part (The value is in unit of TRATE) |

IRCTRL_MF_IR_DEC0_LDR_IDLE

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 28:16 | R/W | 0 | Max duration of Leader’s idle part |
| 12:0 | R/W | 0 | Min duration of Leader’s idle part |

IRCTRL_MF_IR_DEC0_LDR_REPEAT

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 25:16 | R/W | 0 | Max duration of Repeat Code’s Leader. In NECformat, it defines for the repeat leader’s idle part. In Toshiba format, it defines for the repeat leader’s second idle part (In Toshiba format, the repeat leader’s first idle part has the same duration time as the normal leader idle part.) |
| 9:0 | R/W | 0 | Min duration of Repeat Code’s Leader |

IRCTRL_MF_IR_DEC0_BIT_0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 25:16 | R/W | 0 | Max duration of Duration Setting Regiser 0. It defines max tining duration for: Logic”0” for NEC/Toshiba/Sony/Thomas format or Half trailer bit for RC6 format (RC6’s half trailer bit typically 888.89us) or time of Duokan/RCMM/4ppm format’s Logic “00” |
| 9:0 | R/W | 0 | Min duration of Duration Setting Regiser 0. |

IRCTRL_MF_IR_DEC0_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Clock gating control just in case. Set 1 to force clock gating disabled. |
| 30:28 | R/W | 0 | Filter ctrl. Set the monitor timing for input filter, bigger value means longer monitor time. Value 0 = no filtering. |
| 24:12 | R/W | 0 | Max frame time. Max duration of one whole frame. In fact, it is maximum detection window time of a data frame The value of this field is in unit of base time resolution TRATE, so the actual timing duration is : TFRMMAX = Max_frame_time x TRATE. For example, if Max_frame_time is set to decimal 5500, and TRATE= 20uS, then TFRMMAX = 5500 * 20uS = 110000uS = 110mS Note: TFRMMAX should be large than the time between data frame start time and repeat code start time. |
| 11:0 | R/W | 0 | Base time parameter. Used to generate the timing resolution TRATE. Resolution = (base_time_paramter + 1) * (1/ Freq_sys_clk). For example, if Frequency of sys_clk is 1Mhz, and base_time_parameter=19, then the base time resolution TRATE = (19+1)*(1uS) = 20uS. |

IRCTRL_MF_IR_DEC0_FRAME

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | 32 bit Read-Only register stores frame body (LSB 32bit) captured from IR remote data flow, commonly includes custom/address code and data code. |

IRCTRL_MF_IR_DEC0_STATUS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | Frame data valid 1. ( This bit is set to 1 when a captured frame is updated/stored into “FrameBody_1” register. A read of “FrameBody_1” register will clear this bit. “FrameBody_1” register is used to store the over 32bit MSBs of the formats whose length is more than 32 bit ) |
| 30 | R | 0 | bit_1_match_en. Set to 1 to enable the check of whether logic”1” bit matches timing configure during the frame input process. |
| 29:20 | R | 0 | Max Duration 1. Max duration of Duration Setting Regiser 1. It defines max duration for: Logic”1” for NEC/Toshiba/Sony format or Whole trailer bit for RC6 format (RC6’s whole trailer bit typically 1777.78us) or time of Duokan/RCMM/4ppm format’s Logic “01” |
| 19:10 | R | 0 | Min Duration 1. Min duration of Duration Setting Regiser 1. |
| 9 | R | 0 | irq_status. Read Only. |
| 8 | R | 0 | ir_i_sync. IR remote serial input after synchronization. Read Only. |
| 7 | R | 0 | Busy. When =1, means state machine is active. Read Only. |
| 6:4 | R | 0 | Decoder_status. Read Only. |
| 3:0 | R | 0 | Frame status. Read Only. Bit 3: Frame data valid ( This bit is set to 1 when a captured frame is updated/stored into “FrameBody” register. A read of “FrameBody” register will clear this bit. If store and read occurs at the same time, this bit is set to 1 in common, but if “Hold first” is set to true and this valid bit is already 1, a read clear takes precedance and this bit is clear to 0. ) Bit 2: data code error (data != ~data in IR bit stream) Bit 1: custom code error (custom_code != ~custom_code in IR bit stream) Bit 0: 1 = received frame is repeat key, 0 = received frame is normal key |

IRCTRL_MF_IR_DEC0_REG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Set to 1 to use faster timebase. |
| 30 | R/W | 0 | cntl_1us_eq_clk. Just use sys_clk to relace 1uS tick. |
| 29 | R/W | 0 | cntl_xtal3_eq_clk. Just use sys_clk to relace 111ns tick. |
| 28:16 | R/W | 0 | Pulse Width Counter. Read Only fields. It stores the internal counter of pulse width duration. |
| 15 | R/W | 0 | Enable. 1 = enable the state machine of IR decoder. 0 = disable the state machine of IR decoder. |
| 14 | R/W | 0 | cntl_use_sys_clk. Use sys_clk for the timebase. |
| 13:8 | R/W | 0 | Bit_length minus 1. (N-1). Used to set the value of frame body’s bit length (frame body commonly includes address and data code part). If a format has 24 bit frame body, this value shall be set to 23. |
| 7 | R/W | 0 | Record_at_error. 1= record the frame body and status forcibly, even if data/custom code error check enabled by frame_mask and relative error occurs. 0 = if data/custom code error check enabled by frame_mask and relative error occurs, not record the frame body and status forcibly |
| 6 | R/W | 0 | Hold_first Used to hold the first captured frame data. If this bit is set to 1, then the “FrameBody/FrameBody_1” register will only be updated if hasn’t already been updated. Once updated, the “FrameBody/FrameBody_1” register will not be updated again until it has been read. This bit can be used to guarantee the first TV remote code captured will not be overwritten by subsequent transmissions from a TV remote. NOTE: Read the”FrameBody” register can clear the internal “Frame data valid” flag, and read the “FrameBody_1” register can clear the “Frame data valid 1” flag. |
| 5:4 | R/W | 0 | Frame_mask. Some formats’ body include bit-inversed data or custom/address code for error check. 00 = ignore error check from either data or custom/address code 01= check if if data code matches its inverse values, ignore error check from custom/address code 10= check if if custom/adder code matches its inverse values, ignore error check from custom/address code 11= check if data and custom codes match their inverse values |
| 3:2 | R/W | 0 | Irq_sel. IRQ Selection and width measurement reset: 00: IR Decoder done 01: IR input rising or falling edge detected 10: IR input falling edge detected 11: IR rising edge detected |
| 1 | R/W | 0 | IR input polarity selection. Used to adjust/invert the polarity of IR input waveform. |
| 0 | R/W | 0 | Decoder Reset. Set to 1 to reset the IR decoder. This is useful because the IR remote state machine thinks in terms of milliseconds and may take tens of milliseconds to return to idle by itself. |

IRCTRL_MF_IR_DEC0_REG2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 26 | R/W | 0 | Width_low_enable. Enable counter record of low pulse width duration. 0 = do not force enable of width low counter record 1 = force enable of width low counter record Some IR formats’ decoding need to use internal width low counter record. By default, the width low counter record is enabled automatically for related formats. This bit is used for enable forcibly just in case. Besides, if “leader plus stop bit” method is enabled for repeat detection, this bit is also need to be enabled. |
| 25 | R/W | 0 | Width_high_enable. Enable counter record of high pulse width duration. 0 = do not force enable of width high counter record 1 = force enable of width high counter record Some IR formats’ decoding need to use internal width high counter record. By default, the width high counter record is enabled automatically for related formats. This bit is used for enable forcibly just in case. |
| 24 | R/W | 0 | Enable “leader plus stop bit” method for repeat detection . 0 = “leader plus stop bit” method disabled 1 = “leader plus stop bit” method enabled Some IR formats use one normal frame's leader followed by a stop bit to represet repeat. There is no frame data in this kind repeat frame. To use this method, width_low_enable ( bit[26] of 0x20 offset register) shall be set to 1, and max_duration_3 and min_duration_3 in 0x28 offset register shall be set to appropriate value for stop bit's timing duration. |
| 21:16 | R/W | 0 | Repeat_bit_index. These bits are used for compare bit method to set the index of the bit that is used as repeat flag. The index value can be 0 to 63. Compare bit method is one of the methods for repeat detection . Some IR formats use one bit in frame to represet whether the frame is repeat. |
| 15 | R/W | 0 | Running_count_tick_mode. This bit is only valid when use_clock_to_counter bit is 0. 0 = use 100uS as increasing time unit of frame-to-frame counter 1 = use 10uS as increasing time unit of frame-to-frame counter |
| 14 | R/W | 0 | Use_clock_to_counter. If this bit is set to 1, the running_count_tick_mode bit is ignored. 0 = do not use system clock as increasing time unit of frame-to-frame counter 1 = use system clock as increasing time unit of frame-to-frame counter |
| 13 | R/W | 0 | Enable frame-to-frame time counter (running-counter). 0 = frame-to-frame time counter disabled 1 = frame-to-frame time counter enabled If enabled, the frame-to-frame counter increases every 100uS or 10uS until it reaches its max value(all bits are 1) or it is reset. When it reaches its max value, it keeps the value until it is reset. When it is reset, it becomes zero and then begin increasing again. The counter can be reset even when it has not reached its max value. The increasing time unit can be 100uS or 10uS or system clock frequency which is set by running_count_tick_mode and use_clock_to_counter settings. When a frame’s data are capured and stored into FrameBody/FrameBody_1 register, frame-to-frame counter is reset to zero. After reset to zero, the frame-to-frame counter will begin increasing again, until it reaches its max value or it is reset. For repeat frame detection, users can use hardware detection by enabling compare frame or compare bit method, or users can read frame-to-frame counter to let software to make the decision. |
| 12 | R/W | 0 | Enable repeat time check for repeat detection . This bit is valid only when compare frame method or compare bit method is enabled. 0 = repeat time check disabled 1 = repeat time check enabled When repeat frame detection is enabled by enabling compare frame or compare bit method, the frame time interval may need to be checked in order to decide whether the frames are repeat (key pressed without release) or not. You can configure the repeat_time_max value by setting 0x38 offset register. If frame interval is smaller than the “repeat time max”, it may considered as repeat. If frame interval is bigger than the “repeat time max”, it is considered as not repeat. |
| 11 | R/W | 0 | Enable compare frame method for repeat detection . 0 = compare frame method disabled 1 = compare frame method enabled Some IR formats transer the same data frame as repeat frame when the key is kept pressed without release. For repeat detection, compare frame method can be used. If a new frame and the old received frame are the same and the repeat time is under the limit( frame-to-frame time counter value is smaller than the repeat_time_max ), the status register's frame_status[0] is set to 1 automatically as repeat detected flag. You can configure the repeat_time_max value by setting 0x38 offset register. |
| 10 | R/W | 0 | Enable compare bit method for repeat detection . 0 = compare bit method disabled 1 = compare bit method enabled Some IR formats use only one bit to represet whether the frame is repeat. You can compare only one bit instead of compare the whole frame for repeat detection. If compare frame method is enabled, then this bit is ignored. |
| 9 | R/W | 0 | Disable read-clear of FrameBody/FrameBody_1. 0 = read-clear enabled 1 = read-clear disabled FrameBody/FrameBody_1 registers are read-cleared in default. When these register are read, they are cleared to zero. This bit is used to disable this read-clear feature. ( FrameBody/FrameBody_1 registers are used to store captured frame data ). |
| 8 | R/W | 0 | input stream bit order. 0 = LSB first mode (first bit in input stream is considered as LSB) 1= MSB first mode (first bit in input stream is considered as MSB) Note: Commonly the following formats shall set 1 to enable MSB first mode ( unless you insist on LSBfirst mode for your specified use): RC5, RC5 extend, RC6, RCMM, Duokan, Comcast |
| 3:0 | R/W | 0 | Decode_mode.(format selection) 0x0 =NEC 0x1= skip leader (just bits, without leader) 0x2=General time measurement (measure width, software decode) 0x3=MITSUBISHI or 50560 0x4=Thomson 0x5=Toshiba 0x6=Sony SIRC 0x7=RC5 0x8=Reserved 0x9=RC6 0xA=RCMM 0xB=Duokan 0xC=Reserved 0xD=Reserved 0xE=Comcast 0xF=Sanyo |

IRCTRL_MF_IR_DEC0_DURATN2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 25:16 | R/W | 0 | Max duration of Duration Setting Regiser 2. It defines max duration for: Half bit for RC5/6 format (RC5 typically 888.89us for half bit, RC6 typically 444.44us) or time of Duokan/RCMM/4ppm format’s Logic “10” or time of Comcast/16ppm’s base duration |
| 9:0 | R/W | 0 | Min duration of Duration Setting Regiser 2. |

IRCTRL_MF_IR_DEC0_DURATN3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 25:16 | R/W | 0 | Max duration of Duration Setting Regiser 3. It defines max duration for: Whole bit for RC5/6 format (RC5 typically 1777.78us for whole bit, RC6 typically 888.89us) or time of Duokan/RCMM/4ppm format’s Logic “11” or time of Comcast/16ppm’s offset duration |
| 9:0 | R/W | 0 | Min duration of Duration Setting Regiser 3. |

IRCTRL_MF_IR_DEC0_FRAME1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | Stores frame body excess 32 bit range. (MSB 32bit) |

IRCTRL_MF_IR_DEC0_STATUS1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19:0 | R | 0 | Stores the last frame-to-frame counter value before the last counter reset caused by the last frame data record/update. |

IRCTRL_MF_IR_DEC0_STATUS2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19:0 | R | 0 | Stores the value of the frame-to-frame counter which is running currently. |

IRCTRL_MF_IR_DEC0_REG3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19:0 | R/W | 0 | Repeat time max. Used to set the maximum time between two repeat frames for repeat frame detection. Time unit is 100uS or 10uS according to the running_count_tick_mode. When repeat frame detection is enabled by enabling compare frame or compare bit method, the frame time interval may need to be checked in order to decide whether the frames are repeat (key pressed without release) or not. If frame interval is smaller than the “repeat time max”, it may considered as repeat. If frame interval is bigger than the “repeat time max”, it is considered as not repeat. |

IRCTRL_MF_IR_DEC0_FRAME_RSV0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | Frame Reserved register 0. Stores the LSB 32bit of the previous frame’s frame data while the “Frame_Body” register stores the LSB 32bit of the current frame’s frame data. |

IRCTRL_MF_IR_DEC0_FRAME_RSV1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | Frame Reserved register 0. Stores the LSB 32bit of the previous frame’s frame data while the “Frame_Body” register stores the LSB 32bit of the current frame’s frame data. |

IRCTRL_MF_IR_DEC0_FILTE

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | New filter ctrl enable: 1:new filter ctrl enable 0:old filter ctrl enable |
| 12:0 | R/W | 0 | New Filter ctrl Set the monitor timing for input filter, bigger value means longer monitor time. Value 0 = no filtering. |

IRCTRL_MF_IR_DEC0_IRQ_CTL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Interrupt enable 1:remote ir will generate irq 0:remote ir will not generate irq |
| 30 | R/W | 0 | Repeat idle mux 1:not check input single in NEC repeat idle state in the time of repeat_dely_time 0:check single |
| 29 | R/W | 0 | Release status Release pending bit ,write 1 to 0 |
| 28:16 | R/W | 0 | Repeat_dely_time The time will not check input single after repeat leader |
| 15:14 | R/W | 0 | reversed |
| 13 | R/W | 0 | Release irq enable 1:release will generate irq 0:release will not generate irq |
| 12:0 | R/W | 0 | Release_det_time The time to judge release after irq |

IRCTRL_MF_IR_DEC0_FIFO_CTL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Fifo mode enable,only use in measure mode 1:fifo mode enable 2:use old measure mode |
| 30 | R/W | 0 | Fifo trigger pending,write 1 to 0 Fifo have received X dates.X configured by IR_DEC_FIFO_CTL[20:13] |
| 29 | R/W | 0 | Fifo in timeout pending,write 1 to 0 Fifo have not received data in X.X configured by IR_DEC_FIFO_CTL[12:0] |
| 28 | R/W | 0 | Fifo full |
| 27 | R/W | 0 | Fifo empty |
| 26 | R/W | 0 | Fifo initial |
| 25:21 | R/W | 0 | revered |
| 20:13 | R/W | 0 | Fifo trigger irq level Default is ‘h16 |
| 12:0 | R/W | 0 | Fifo in timeout Default is ‘d800 |

IRCTRL_MF_IR_DEC0_WIDTH_NEW

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 12:0 | R/W | 0 | Width new When use fifo mode,should read width from this register |

IRCTRL_MF_IR_DEC0_REPEAT_DET

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Repeat detect mux enable |
| 28:16 | R/W | 0 | Max repeat idle time |
| 12:0 | R/W | 0 | Min repeat idle time |

IRCTRL_MF_IR_DEC1/2/3_* register description are same as IRCTRL_MF_IR_DEC0_*

IRCTRL_IR_DEC_DEMOD0_CNTL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Ir demodulor soft_reset:write 1 to reset ir_demodulor,will auto clr to 0 |
| 30 | R/W | 0 | Reg_ir_fd_reset :It is used to reset the carrier detection module.write 1 to reset ,will auto clr to 0 |
| 29 | R/W | 0 | Reg_ir_demod_mode:It is uset to set the demod mode. 0 = count mode; 1 = iir mode. |
| 28 | R/W | 0 | Ir demodulor clk gate bypass:write 1 will bypass clk gate,default is 0 |
| 27:16 | R/W | 0x40 | Reg_ir_st_cnt_thd:It is used to set the statistics number for carrier detection. |
| 15:8 | R/W | 8 | Reg_ir_ds_rate:It is uset to set the downsample rate. |
| 7:4 | R/W | 7 | Reg_ir_fsft_1:It is used to set the shift value for input data “1”. |
| 3:0 | R/W | 9 | Reg_ir_fsft_0:It is used to set the shift value for input data “0”. |

IRCTRL_IR_DEC_DEMOD0_CNTL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Reg_ir_demod_en:ir demod enable , |
| 30 | R/W | 0 | Reg_ir_invt:ir input invert, invert input at 1. |
| 29:16 | R/W | 0xa | Reg_ir_proctect1:It is used to set the protection threshold for 1. It is used to filter the glitch. |
| 13:0 | R/W | 0x3e8 | Reg_ir_detect0:It is used to set the detection threshold for signal “0”. |

IRCTRL_IR_DEC_DEMOD0_IIR_THD

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R/W | 0xdac | Reg_ir_iir_thd1:It is used to set the detection threshold for “1” in iir mode. |
| 15:0 | R/W | 0xdac | Reg_ir_iir_thd0:It is used to set the detection threshold for “0” in iir mode. |

IRCTRL_IR_DEC_DEMOD0_THD0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 29:16 | R/W | 0x12c | Reg_ir_thd0_low:It is used to set the low threshold for “0” when statistics. |
| 13:0 | R/W | 0x1770 | Reg_ir_thd0_high:It is used to set the high threshold for “0” when statistics. |

IRCTRL_IR_DEC_DEMOD0_THD1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 29:16 | R/W | 0x12c | Reg_ir_thd1_low:It is used to set the low threshold for “1” when statistics. |
| 13:0 | R/W | 0x1770 | Reg_ir_thd1_high:It is used to set the high threshold for “1” when statistics. |

IRCTRL_IR_DEC_DEMOD0_SUM_CNT0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 25:0 | R | 0 | Ro_ir_sum_cnt0:It is used to report the sum value for the statistics data “0”. |

IRCTRL_IR_DEC_DEMOD0_SUM_CNT1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 25:0 | R | 0 | Ro_ir_sum_cnt1:It is used to report the sum value for the statistics data “1” |

IRCTRL_IR_DEC_DEMOD0_CNT0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 29:16 | R | 0 | Ro_ir_max_cnt0:It is used to report the maximum value for the statistics data “0”. |
| 13:0 | R | 0 | Ro_ir_min_cnt0:It is used to report the minimum value for the statistics data “0”. |

IRCTRL_IR_DEC_DEMOD0_CNT1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 29:16 | R | 0 | Ro_ir_max_cnt1:It is used to report the maximum value for the statistics data “1”. |
| 13:0 | R | 0 | Ro_ir_min_cnt1:It is used to report the minimum value for the statistics data “1”. |

IRCTRL_IR_DEC_DEMOD1_* register description are same as IRCTRL_IR_DEC_DEMOD0_*

IRCTRL_IR_BLASTER_ADDR0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-27 | R | 0 | unused |
| 26 | R | - | BUSY: If this bit is 1, then the IR Blaster module is busy. |
| 25 | R | - | This output is 1 when the FIFO is Full |
| 24 | R | - | This output is 1 when the FIFO is Empty |
| 23-16 | R | - | FIFO Level |
| 15-14 | R/W | 0 | Unused |
| 13-12 | R/W | 0 | MODULATOR_TB: This input controls the clock used to create the modulator output. The modulator is typically run between 32khz and 56khz. The modulator output will equal a divided value of the following: 00: system clock “clk” 01: mpeg_xtal3_tick 10: mpeg_1uS_tick 11: mpeg_10uS_tick |
| 11-4 | R/W | 0 | SLOW_CLOCK_DIV: This is a divider value used to divide down the input “clk”. The divider is N+1 so a value of 0 equals divide by 1. |
| 3 | R/W | 0 | SLOW_CLOCK_MODE: Set this signal high to use a special mode in which the “clk” input is driven by a slow clock less than 1Mhz. This is used for low power cases where we want to run the IR Blaster between 32khz and 1Mhz |
| 2 | R/W | 0 | INIT_LOW: Setting this bit to 1 initializes the output to be high. Please set this bit back to 0 when done |
| 1 | R/W | 0 | INIT_LOW: Setting this bit to 1 initializes the output to be low. Please set this bit back to 0 when done |
| 0 | R/W | 0 | ENABLE: 1 = Enable. If this bit is set to 0, then the IR blaster module is reset and put into an IDLE state. |

IRCTRL_IR_BLASTER_ADDR1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-28 | R/W | 0 | unused |
| 27-16 | R/W | 0 | This value is used with “modulator_tb[1:0]” above to create a low pulse. The time is computed as (mod_lo_count+1) x modulator_tb. The purpose for having a low/high count is the modulator output might not be 50% duty cycle. Hi/Lo counters allow us to modulate using a non-50% duty cycle waveform. |
| 15-12 | R/W | 0 | Unused |
| 11-0 | R/W | 0 | This value is used with “modulator_tb[1:0]” above to create a high pulse. The time is computed as (mod_hi_count+1) x modulator_tb |

IRCTRL_IR_BLASTER_ADDR2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-17 | R | 0 | unused |
| 16 | W | 0 | Set this bit to 1 to write the data below to the FIFO |
| 15-12 | R | 0 | Unused |
| 11-0 | R/W | 0 | FIFO data to be written: Bit[12] output level (or modulation enable/disable: 1 = enable) Bit[11:10] Timebase: 00 = 1uS 01 = 10uS 10 = 100uS 11 = Modulator clock Bit[9:0] Count of timebase units to delay |

IRCTRL_IR_BLASTER_ADDR3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-17 | R | 0 | unused |
| 16 | R | 0 | fifo thd pending |
| 8 | R/W | 0 | FIFO irq enable |
| 7:0 | R/W | 0 | FIFO irq threshold. |

- UART_A/B/

B are the same design as A:

UART_A_WFIFO

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-8 | R | 0 | unused |
| 7-0 | R/W | - | Write FIFO data. The Write FIFO holds 128 or 64 bytes. The Write FIFO can be written as long as it is not full. |

UART_A_RFIFO

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-8 | R | 0 | unused |
| 7-0 | R/W | - | Read FIFO data. The Read FIFO holds 128 or 64 bytes. The empty flag can be used to determine if data is available |

UART_A_CONTROL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Invert the RTS signal |
| 30 | R/W | 0 | Mask Error: Set to 1 to mask errors |
| 29 | R/W | 0 | Invert the CTS signal |
| 28 | R/W | 0 | Transmit byte Interrupt: Set to 1 to enable the generation an interrupt whenever a byte is read from the transmit FIFO |
| 27 | R/W | 0 | Receive byte Interrupt: Set to 1 to enable the generation an interrupt whenever a byte is written to the receive FIFO |
| 26 | R/W | 0 | Set to 1 to invert the TX pin |
| 25 | R/W | 0 | Set to 1 to invert the RX pin |
| 24 | R/W | 0 | Clear Error |
| 23 | R/W | 0 | Reset the receive state machine |
| 22 | R/W | 0 | Reset the transmit state machine |
| 21-20 | R/W | 0 | Character length: 00 = 8 bits, 01 = 7 bits, 10 = 6 bits, 11 = 5 bits |
| 19 | R/W | 1 | Parity Enable: Set to 1 to enable parity |
| 18 | R/W | 0 | Parity type: 0 = even, 1 = odd |
| 17-16 | R/W | 0 | Stop bit length: 00 = 1 bit, 01 = 2 bits |
| 15 | R/W | 0 | Two Wire mode: |
| 14 | R/W | 0 | Unused |
| 13 | R/W | 0 | Receive Enable. Set to 1 to enable the UART receive function |
| 12 | R/W | 0 | Transmit Enable. Set to 1 to enable the UART transmit function |
| 11-0 | R/W | 0x120 | Old Baud rate: |

UART_A_STATUS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-27 | R | 0 | Unused |
| 26 | R | 0 | UART_RECV_BUSY: This bit will be 1 if the uart receive state machine is busy |
| 25 | R | 0 | UART_XMIT_BUSY: This bit will be 1 if the uart transmit state machine is busy |
| 24 | R | 0 | RECV_FIFO_OVERFLOW: |
| 23 | R | 0 | CTS Level |
| 22 | R | 0 | Transmit FIFO Empty |
| 21 | R | 0 | Transmit FIFO Full |
| 20 | R | 0 | Receive FIFO empty |
| 19 | R | 0 | Receive FIFO full |
| 18 | R | 0 | This bit is set if the FIFO is written when it is full. To clear this bit, write bit 24 of register 0x2132 |
| 17 | R | 0 | Frame error. To clear this bit, write bit 24 of register UART0_CONTROL |
| 16 | R | 0 | Parity error. To clear this bit, write bit 24 of register UART0_CONTROL |
| 15 | R | 0 | Unused |
| 14-8 | R | 0 | Transmit FIFO count. Number of bytes in the transmit FIFO |
| 7 | R | 0 | Unused |
| 6-0 | R | 0 | Receive FIFO count. Number of bytes in the receive FIFO |

UART_A_MISC

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Added a “just in case” bit that can be set to 1 to enable clocks always. The default is 0 meaning the auto-clock gating logic is enabled. |
| 30 | R/W | 0 | USE old Rx Baud: There was a bug in the RX baud rate generator. The Rx baud rate generator was re-designed to compute a baud rate correctly. If you want to use the old (stupid) logic, you can set this bit to 1. |
| 29 | R/W | 0 | ASYNC_FIFO_PURGE: This bit can be set to 1 after all UART bytes have been received in order to purge the data into the Async FIFO. This bit is needed because the UART receives 8-bit data, but the ASYNC FIFO can only be written with 16-bit data. In this case there might be a residual byte if the UART is not receiving an even number of bytes. |
| 28 | R/W | 0 | ASYNC_FIFO_EN: If this bit is set to 1, then the UART received data is automatically sent to the Async FIFO module which will in turn automatically send the data to DDR memory |
| 27 | R/W | 0 | CTS: Filter Timebase select: 1 = 1uS, 0 = 111nS timebase. A filter was added to the CTS input to allow for a little digital filtering. The amount of filtering is controlled by this timebase (longer = more filtering) and the value in bits FILTER_SEL below. |
| 26-24 | R/W | 0 | CTS: FILTER_SEL: 0 = no filter, 7 = max filtering |
| 23-20 | R/W | 0 | Old BAUD_RATE_EXT: These 4 bits extend the baud rate divider to 16-bits: Baud Rate = {Reg4[23:20],Reg2[11:0]} |
| 19 | R/W | 0 | RX: Filter Timebase select: 1 = 1uS, 0 = 111nS timebase. A filter was added to the RX input to allow for a little digital filtering. The amount of filtering is controlled by this timebase (longer = more filtering) and the value in bits FILTER_SEL below. |
| 18-16 | R/W | 0 | RX: FILTER_SEL: 0 = no filter, 7 = max filtering |
| 15-8 | R/W | 32 | XMIT_IRQ_CNT: The UART can be configured to generate an interrupt if the number of bytes in the transmit FIFO drops below this value. |
| 7:0 | R/W | 15 | RECV_IRQ_CNT: The UART can be configured to generate an interrupt after a certain number of bytes have been received by the UART. |

UART_A_REG5

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-28 | R/W | 0 | unused |
| 27 | R/W | 0 | Xtal2_clk_sel: 0: see Xtal_clk_sel 1: xtal_div2(12M) |
| 26 | R/W | 0 | Xtal_clk_sel: 0: xtal_div3(8M); 1: xtal(24M);(need set xtal_tick_en =1 first); |
| 24 | R/W | 0 | USE_XTAL_CLK: If this bit is set, then the clock for generating the UART Baud rate comes from the crystal pad. This allows the UART to operate independent of clk81. |
| 23 | R/W | 0 | USE New Baud rate. Over the years, the baud rate has been extended by concatenating bits from different registers. To take advantage of the full 23-bit baud rate generate (extended to 23 bits to accommodate very low baud rates), you must set this bit. If this bit is set, then the baud rate is configured using bits [22:0] below |
| 22:0 | R/W | 15 | NEW_BAUD_RATE: If bit[23] = 1 above, then the baud rate for the UART is computed using these bits. |

- I2C_A/B/C

B/C/ are the same design as A:

Register Map

| Address | Name | Note |
| --- | --- | --- |
| BASE + 0x0 | RDY | Ready and tee |
| BASE + 0x4 | I2C | Set DMA and join |
| BASE + 0x8 | Start | Write at last |
| BASE + 0xC | Bus | Set slave mode |
| BASE + 0x10 | tx_rd_addr | tx fifo pull address |
| BASE + 0x14 | tx_wr_addr | tx fifo push address |
| BASE + 0x18 | rx_rd_addr | rx fifo pull address |
| BASE + 0x1C | rx_wr_addr | rx fifo push address |
| BASE + 0x20 | tx | empty, full, tx_data |
| BASE + 0x24 | rx | empty, full, wch_data |
| BASE + 0x30 | Irq_sts | Write 1 to clear irq |
| BASE + 0x34 | Irq_enable | Irq mask |
| BASE + 0x38 | shake | RO |
| BASE + 0b1xxxxxxx | FIFO addr | Set fifo use APB mode |

CFG_RDY

BASE + 0

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 1 | Tee_open | If 1, tee open, everyone can use; If 0, tee_lock, just PUSER[9:2] = ‘b11 can use. Default 0. | New feature in r0.4 |
| 0 | I2c_rdy | if 0, controller own i2c | Not mandatory, for sw use |

CFG_I2C

BASE + 4

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 23:16 | rx_thresh | rx_fifo_irq = rx_count >= rx_thresh | for software, verificaition don’t care |
| 15:8 | tx_thresh | tx_fifo_irq = tx_count <= tx_thresh | for software, verificaition don’t care |
| 7:5 | RESV |  |  |
| 4 | b_dma | set 1 to use DMA | only in DMA mode |
| 3 | b_hw_neg | use hw_trig’s negedge to start | only in hw_trig mode |
| 2 | b_hw_pos | use hw_trig’s posedge to start | only in hw_trig mode |
| 1 | b_join_rx | if 1, the whole fifo is used by RX | strech rx fifo |
| 0 | b_join_tx | if 1, the whole fifo is used by TX | strech tx fifo |

CFG_START

Base + 8

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 31 | b_pending | set 1 to start trans |  |
| 30:24 | RESV |  |  |
| 23:12 | b_length | number of blocks | if 0, the number is 2^12 |
| 11 | b_10bits | 10 bits slave addr mode | reserved for future update |
| 10:1 | b_slave_addr | 7bits or 10 bits | this version just support 7bits |
| 0 | b_mode | transfer mode | 0: write; 1: read |

CFG_BUS

BASE + 0xC

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 17 | b_speed_mode | 0: other mode; 1: 100k | Fix su/hd timing in T6X/C4 |
| 16 | b_slave_mode | 1: I2C slave mode | Default 0: master |
| 15:12 | b_filter_sel | used in din_filter to avoid glitch | dout is delayed (b_filter_sel + 2) sys_clk by din |
| 11:0 | b_ratio | SCL = clk/b_ratio | if b_ratio<=8, SCL = clk/8 |

TX&RX ADDR

BASE +

| ADDR | Name | Details | Note |
| --- | --- | --- | --- |
| 10 | TX_RD_ADDR | tx fifo pull address |  |
| 14 | TX_WR_ADDR | tx fifo push address |  |
| 18 | RX_RD_ADDR | rx fifo pull address |  |
| 1C | RX_WR_ADDR | rx fifo push address |  |

CFG_TX/RX

BASE + 0x20/24

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 9 | empty | Tx/rx fifo empty | RO |
| 8 | full | Tx/rx fifo full | RO |
| 7:0 | data | Tx_data/wch_data | Tx data W/R, wch_data data RO |

IRQ_STS/IRQ_ENABLE

BASE + 0x30 / BASE + 0x34

| Bits | Name | Details |
| --- | --- | --- |
| 7 | phy_done | transfer done |
| 6 | tx_fifo_irq | for software (verification don’t care this feature) |
| 5 | rx_fifo_irq | for software (verification don’t case this feature) |
| 4 | apb_tx_err | tx_full |
| 3 | apb_rx_err | rx_empty |
| 2 | phy_err | NACK error |
| 1 | wch_err | wch data transfer error |
| 0 | rch_err | rch data transfer error |

IRQ_STS always updated by hardware.

SHAKE & BLK_CNT

BASE + 0x38

| Bits | Name | Details | Note |
| --- | --- | --- | --- |
| 16:4 | Blk_cnt | Number of Bytes | RO |
| 3 | tx_data_vld |  |  |
| 2 | tx_data_rdy |  |  |
| 1 | rx_data_vld |  |  |
| 0 | rx_data_rdy |  |  |

ACPU IRQ is only trigger if IRQ_ENABLE bit is set to 1.

FIFO WRITE

BASE + 0b1xxxxxxx

| Bits | Name | Note |
| --- | --- | --- |
| 31:24 | data_fifo[fifo_adr +3] | when fifo_wr == 1, use APB to set FIFO. fifo_wr = PADDR[7]==1, fifo_adr = PADDR[6:0] |
| 23:16 | data_fifo[fifo_adr +2] |  |
| 15:8 | data_fifo[fifo_adr +1] |  |
| 7:0 | data_fifo[fifo_adr +0] |  |

- PWM_A/B/C/D/E/F/G/H

Each PWM module can generate 1 PWM.

so PWM_B/C/D/E/F/G/H usage are the same as PWM_A:

PWM_PWM_A

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-15 | R/W | 134 | PWM_A_HIGH: This sets the high time (in clock counts) for the PWM_A generator output |
| 15-0 | R/W | 134 | PWM_A_LOW: This sets the high time (in clock counts) for the PWM_A generator output |

PWM_MISC_REG_A

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 30 | R/W | 0 | pwm_A_hiz when hiz mode, pwm_o will connect to pad_oe, then pad_o will equal this bit |
| 28 | R/W | 0 | pwm_A_constant_en |
| 26 | R/W | 0 | pwm_A_inv_en |
| 25 | R/W | 0 | cntl_pwm_a2_en |
| 2 | R/W | 0 | DS_A_EN: This bit is only valid if PWM_A_EN is 0: if this bit is set to 1, then the PWM_A output is configured to generate a delta sigma output based on the settings in the register below. If this bit is set to 0, then the PWM_A output is set low. |
| 0 | R/W | 0 | PWM_A_EN: If this bit is set to 1, then the PWM_A output is configured to generate a PWM output based on the register above. If this bit is 0, then the PWM_A output is controlled by DS_A_EN above. |

PWM_DELTA_SIGMA_A

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15-0 | R/W | 0x8000 | DS_A_VAL: This value represents the delta sigma setting for channel A (PWM_A) |

PWM_TIME_A

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-24 | R/W | 10 | A_timer1 |
| 23:16 | R/W | 10 | A_timer2 |

PWM_A2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-15 | R/W | 134 | PWM_A2_HIGH: This sets the high time (in clock counts) for the PWM_A generator output |
| 15-0 | R/W | 134 | PWM_A2_LOW: This sets the high time (in clock counts) for the PWM_A generator output |

PWM_BLINK_A

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-10 | R | 0 | Reserved |
| 8 | R/W | 0 | blink enable for pwm A |
| 3-0 | R/W | 0 | blink times for pwm A |

PWM_LOCK_A

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 0 | 1: will lock all PWM register |

PWM_TEE_ONLY_A

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 0 | pwm tee only lock |

- SPISG_0

Base+0x0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |

Base+0x4

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| [30:27] |  | 0 | add 4 high bits in gap_cdn |
| [26:12] |  | 0 | number of clk_spi, for delay SS up |
| [11:10] |  | 0 | config MOSI_o’s level in IDLE time |
| [9:8] |  | 0 | number of sclk between 2 blocks |
| [7] |  | 0 | Start desc on HW falling edge |
| [6] |  | 0 | Start desc on HW rising edge |
| [5] |  | 0 | SPI flash hold |
| [4] |  | 0 | SPI flash WP |
| [3:2] |  | 0 | Slave Select |
| [1] |  | 0 | Slave mode |
| [0] |  | 0 | Bus64 |

CFG_START Base+0x8

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 |  | 0 | Pending,Set to 1 to start Cleared to 0 by SPICC |
| 30 |  | 0 | End of chain |
| 29:28 |  | 0 | TX mode,Data source |
| 27:26 |  | 0 | RX mode,Data destination |
| 25:24 |  | 0 | Operation mode |
| 23 |  | 0 | Data or command |
| 22:20 |  | 0 | Number of bytes |
| 19:0 |  | 0 | Number of blocks |

CFG_BUS Base+0xC

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 |  | 0 | CPOL,SCLK polarity |
| 30 |  | 0 | CPHA,SCLK phase |
| 29 |  | 0 | Keep slave selected |
| 28:27 |  | 0 | Dummy SCLK |
| 26 |  | 0 | Send SCLK in gap after SS |
| 24 |  | 0 | DC mode,9bit or pin |
| 23 |  | 0 | Endian 0: big ( MSByte ... LSByte ) 1: little ( LSByte … MSbyte ) |
| 22 |  | 0 | Half duplex,I/O using the same PIN |
| 21:20 |  | 0 | Lane 0: Single SPI 1: Dual SPI 2: Quad SPI 3: reserved |
| 19:16 |  | 0 | Wait for slave, low 4 bits in gap_cnd |
| 15:12 |  | 0 | TX Timing adjust |
| 11:8 |  | 0 | RX Timing adjust |
| 7:0 |  | 0 | Clock SPI to SCLK ratio,SCLK frequency |

Base+0x10

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 |  |  | TX descriptor[31:0],Using sd_emmc link |

Base+0x14

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 |  |  | TX descriptor[63:32],Using sd_emmc link |

Base+0x18

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 |  |  | RX descriptor[31:0],Using sd_emmc link |

Base+0x1C

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 |  |  | RX descriptor[63:32],Using sd_emmc link |

Base+0x20

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 |  |  | Descriptor list[31:0] |

Base+0x24

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 |  |  | Descriptor list[39],Pending |
| 7:0 |  |  | Descriptor list[38:32],8 bits for bus64 |

Base+0x28

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 |  |  | Descriptor Current 0 |

Base+0x2c

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 |  |  | Descriptor Current 1 |

Base+0x30

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 |  |  | IRQ STS |

Base+0x34

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 |  |  | IRQ ENABLE |

| 24 | R/W | 0 | spi clk select 0: controlled by data_rate in CONREG 1: controlled by enhance_clk_div in ENHANCE_CNTL |
| --- | --- | --- | --- |
| 23:16 | R/W | 0 | enhance_clk_div: if bit[24] is 1, sclk will be divided by system clock with 2*enhance_clk_div, Example: if system clock = 400MHz and enhance_clk_div=5, sclk’s frequency equals 40MHz |
| 15:0 | R/W | 0 | clk-cs delay value: when CS active, wait clk-cs cycles before data transfer beginning. Only use in master mode |

- MSR_CLK

clock source as blow:

MSR_CLK_DUTY

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-16 | R | 0 | DUTY_CNT_HIGH: This value represents the HIGH-count during duty cycle measurements |
| 15-0 | R | 0 | DUTY_CNT_LOW: This value represents the LOW-count during duty cycle measurements |

MSR_CLK_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | BUSY: If this bit is set to 1, then the measure logic is busy |
| 30-29 | R | 0 | PROCESS_COUNT: This is a counter that simply increments every time a measurement is started |
| 28 | R | 0 | Limit Error |
| 27-20 | R/W | 0 | MSR_CLK_MUX_SEL: Selects the clock to measure. See example C-code |
| 19 | R/W | 0 | CLK_EN: The incoming clock is gated to prevent glitches from entering the measurement logic. Set this bit to 1 once the external clock has been established and is stable. |
| 18 | RW | 0 | One_shot_enable |
| 17 | R/W | 0 | CONT_EN: Set this bit to 1 to enable continuous frequency measurements |
| 16 | R/W | 0 | ENABLE: This is a general enable to enable the clock measurement circuit. If CONT_EN is set to 1 above, then the circuit will continually measure. If CONT_EN is set to 0, then the circuit will make one measurement. To restart another measurement, you must set ENABLE low then high. |
| 15-0 | R/W | 0 | MSR_TIME: This value establishes the gate time of the measurement with a 1uS resolution. The measurement module will count I2S MCLK clocks for N uS |

MSR_CLK_REG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | DCYCLE_MSR_DONE: If this bit is set to 1, then the duty cycle measurement is done |
| 30 | R/W | 0 | unused |
| 29 | R/W | 0 | DCYCLE_ENABLE: Set this bit to enable start a duty cycle measurement |
| 28 | R/W | 0 | DCYCLE_CLK_EN: Set this bit to enable the high-frequency duty cycle clock |
| 27-20 | R/W | 0 | DCYCLE_TCNT: This value represents the number of uS to measure. This value should be less than 252. |
| 19-0 | R/W | 0 | unused |

MSR_CLK_REG2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19-0 | R | 0 | MEASURED_VALUE: This is the count of cycles measured during the programmed gate time. |

MSR_CLK_REG3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19-0 | R | 0 | Min_exp |

MSR_CLK_REG4

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19-0 | R | 0 | Max_exp |

MSR_CLK_REG5

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R | 0 | Start_p |

- SAR_ADC

SAR_ADC_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | PANEL DETECT level. |
| 30 | R/W | 0 | DELTA_BUSY: If this bit is 1, then it indicates the delta processing engine is busy |
| 29 | R/W | 0 | AVG_BUSY: If this bit is 1, then it indicates the averaging engine is busy |
| 28 | R/W | 0 | SAMPLE_BUSY: If this bit is 1, then it indicates the sampling engine is busy |
| 27 | R/W | 0 | FIFO_FULL: |
| 26 | R/W | 0 | FIFO_EMPTY: |
| 25-21 | R/W | 4 | FIFO_COUNT: Current count of samples in the acquisition FIFO |
| 20-19 | R/W | 0 | ADC_BIAS_CTRL |
| 18-16 | R/W | 0 | CURR_CHAN_ID: These bits represent the current channel (0..7) that is being sampled. |
| 15 | R/W | 0 | unused |
| 14 | R/W | 0 | SAMPLING_STOP: This bit can be used to cleanly stop the sampling process in the event that continuous sampling is enabled. To stop sampling, simply set this bit and wait for all processing modules to no longer indicate that they are busy. |
| 13-12 | R/W | 0 | CHAN_DELTA_EN: There are two bits corresponding to Channels 0 and 1. Channel 0 and channel 1 can be individually enabled to take advantage of the delta processing module. |
| 11 | R/W | 0 | Unused |
| 10 | R/W | 0 | DETECT_IRQ_POL: This bit sets the polarity of the detect signal. The detect signal is used during X/Y panel applications to detect if the panel is touched |
| 9 | R/W | 0 | DETECT_IRQ_EN: If this bit is set to 1, then an interrupt will be generated if the DETECT signal is low/high. The polarity is set in the bit above. |
| 8-4 | R/W | 0 | FIFO_CNT_IRQ: When the FIFO contains N samples, then generate an interrupt (if bit 3 is set below). |
| 3 | R/W | 0 | FIFO_IRQ_EN: Set this bit to 1 to enable an IRQ when the acquisition FIFO reaches a certain level. |
| 2 | W | 0 | SAMPLE_START: This bit should be written to 1 to start sampling. |
| 1 | R/W | 0 | CONTINUOUS_EN: If this bit is set to 1, then the channel list will be continually processed |
| 0 | R/W | 0 | SAMPLING_ENABLE: Setting this bit to ‘1’ enables the touch panel controller sampling engine, averaging module, XY processing engine and the FIFO. |

SAR_ADC_CHAN_LIST

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-27 | R/W | 0 | unused |
| 26-24 | R/W | 2 | Length of the list of channels to process. If this value is 2, then only channels in bits [8:0] below are processed. |
| 23-21 | R/W | 7 | 8th channel |
| 20-18 | R/W | 6 | 7th channel |
| 17-15 | R/W | 5 | 6th channel |
| 14-12 | R/W | 4 | 5th channel |
| 11-9 | R/W | 3 | 4th channel |
| 8-6 | R/W | 2 | 3rd channel |
| 5-3 | R/W | 1 | 2nd channel |
| 2-0 | R/W | 0 | First channel in the list of channels to process |

SAR_ADC_AVG_CNTL

Each channel listed in the CHANNEL_LIST is given independent control of the number of samples to acquire and averaging mode

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-30 | R/W | 0 | Channel 7: Averaging mode: 0 = no averaging, 1 = simple averaging, 2 = median averaging. |
| 29-28 | R/W | 0 | Channel 6: Averaging mode: 0 = no averaging, 1 = simple averaging, 2 = median averaging. |
| 27-26 | R/W | 0 | Channel 5: Averaging mode: 0 = no averaging, 1 = simple averaging, 2 = median averaging. |
| 25-24 | R/W | 0 | Channel 4: Averaging mode: 0 = no averaging, 1 = simple averaging, 2 = median averaging. |
| 23-22 | R/W | 0 | Channel 3: Averaging mode: 0 = no averaging, 1 = simple averaging, 2 = median averaging. |
| 21-20 | R/W | 0 | Channel 2: Averaging mode: 0 = no averaging, 1 = simple averaging, 2 = median averaging. |
| 19-18 | R/W | 0 | Channel 1: Averaging mode: 0 = no averaging, 1 = simple averaging, 2 = median averaging. |
| 17-16 | R/W | 0 | Channel 0: Averaging mode: 0 = no averaging. 1 = simple averaging of the number of samples acquired (1,2,4 or 8). 2 = median averaging. NOTE: If these bits are set to 2, then you must set the number of samples to acquire below to 8. |
| 15-13 | R/W | 0 | Channel 7: Number of samples to acquire 2N: |
| 13-12 | R/W | 0 | Channel 6: Number of samples to acquire 2N: |
| 11-10 | R/W | 0 | Channel 5: Number of samples to acquire 2N: |
| 9-8 | R/W | 0 | Channel 4: Number of samples to acquire 2N: |
| 7-6 | R/W | 0 | Channel 3: Number of samples to acquire 2N: |
| 5-4 | R/W | 0 | Channel 2: Number of samples to acquire 2N: |
| 3-2 | R/W | 0 | Channel 1: Number of samples to acquire 2N: |
| 1-0 | R/W | 0 | Channel 0: Number of samples to acquire 2N: 0 = 1, 1 = 2, 2 = 4, 4 = 8. |

SAR_ADC_REG3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 1 | CNTL_USE_SC_DLY: hold time delay was added to the start conversion clock. Unfortunately, it appears that the analog ADC design requires that we use the inverted clock so this bit is meaningless. |
| 30 | R/W | 0 | reserved |
| 29 | R/W | 0 | Auxsar_div7_sel:0:clk=oscin 1: clk=135M |
| 28 | R/W | 0 | Auxsar_div7_en:clk divide enable when diseqc function |
| 27 | R/W | 1 | saradc_rng_en |
| 26 | R/W | 0 | saradc_diff_en |
| 25~23 | R/W | 0 | SARADC_CTRL[2:0]: |
| 22 | R/W | 0 | reserved |
| 21 | R/W | 0 | ADC_EN: Set this bit to 1 to enable the ADC |
| 20-15 | R/W | 0 | reserved |
| 14 | R/W | 0 | cntl_block_dly_use_sys_clk: 1 block delay counter use sys clk; 0 block delay counter use block delay tick |
| 13 | R/W | 0 | cntl_sample_dly_use_sys_clk: 1 sample delay counter use sys clk; 0 sample delay counter use sample delay tick |
| 12 | R/W | 0 | cntl_input_dly_use_sys_clk: 1 input delay counter use sys clk; 0 input delay counter use input delay tick |
| 9-8 | R/W | 1 | BLOCK_DLY_SEL: when cntl_block_dly_use_sys_clk=0 0 = count 1uS ticks, 1 = count 10uS ticks, 2 = count 100uS ticks. 3 = count 1mS ticks |
| 7-0 | R/W | 8’ha | BLOCK_DLY: After all channels in the CHANNEL_LIST have been processed, the sampling engine will delay for an amount of time before re-processing the CHANNEL_LIST again. Combined with bits [9:8] above, this value is used to generate a delay between processing blocks of channels. |

SAR_ADC_DELAY

As the CHANNEL_LIST is process, the input switches are set according to the requirements of the channel.  After setting the switches there is a programmable delay before sampling begins.  Additionally, each channel specifies the number of samples for that particular channel.  The sampling rate is programmed below.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-29 | R | 0 | unused |
| 28 | R/W | 0 | CNTL_EOC_BY_CNT: ADC dout valid controlled by counter |
| 27 | R/W | 0 | CNTL_USE_LATCHED_DATA: ADC dout be latched first, then be sampled |
| 26 | R | 0 | unused |
| 25-24 | R/W | 0 | INPUT_DLY_SEL: 0 = 111nS ticks, 1 = count 1uS ticks, 2 = count 10uS ticks, 3 = count 100uS ticks |
| 23-16 | R/W | 3 | INPUT_DLY_CNY: For channels that acquire 2,4 or 8 samples, the delay between two samples is controlled by this count (N+1) combined with the delay selection in the two bits above. |
| 15-10 | R/W | 14 | CNTL_EOC_DLY_CNT: the delay between SC and ADC output data ready for latch/sample |
| 9-8 | R/W | 0 | SAMPLE_DLY_SEL: 0 = count 1uS ticks, 1 = count 10uS ticks, 2 = count 100uS ticks. 3 = count 1mS ticks |
| 7-0 | R/W | 9 | SAMPLE_DLY_CNY: For channels that acquire 2,4 or 8 samples, the delay between two samples is controlled by this count (N+1) combined with the delay selection in the two bits above. |

SAR_ADC_LAST_RD

For channel 0 and channel 1, (the special X/Y channels) the last sample pushed into the FIFO for each channel is saved in a register.  This allows the software to see the last sample for channel 0 and channel 1 even when the FIFO overflows.  For example, if we are sampling quickly and there is a gesture on the screen, we can use the contents of the FIFO to see the direction of the gesture and use the last sample values to see where the pen finally came to rest.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-28 | R | 0 | unused |
| 27-16 | R | 0 | LAST_CHANNEL1 |
| 15-12 | R | 0 | unused |
| 11-0 | R | 0 | LAST_CHANNEL0 |

SAR_ADC_FIFO_RD

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-28 | R | 0 | Unused |
| 27-16 | R | 0 | RAW data |
| 15 | R | 0 | Unused |
| 14-12 | R | 0 | Channel ID. This value identifies the channel associated with the data in bits [11:0] below |
| 11-0 | R | 0 | Sample value: 12-bit raw or averaged ADC sample written to the FIFO. |

SAR_ADC_AUX_SW

Channels 2 ~ 7 can program the ADC input mux to any selection between 0 and 7.  This register allows the software to associate a mux selection with a particular channel.  In addition to the ADC mux, there are a number of switches that can be set in any particular state.  Channels 2 ~ 7 share a common switch setting.  Channels 0 and 1 on the other hand have programmable switch settings (see other registers below).

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | EOC |
| 30 | R/W | 0 | CNTL_MANUAL_SC |
| 29 | R/W | 0 | CNTL_MANUAL_CLK |
| 28 | R/W | 0 | CNTL_MANUAL_MODE |
| 27 | R/W | 0 | CNTL_FT_MODE_SCK |
| 26 | R | 0 | unused |
| 25-23 | R/W | 7 | Channel 7 ADC_MUX setting when channel 7 is being measured. |
| 22-20 | R/W | 7 | Channel 6 ADC_MUX setting when channel 6 is being measured. |
| 19-17 | R/W | 7 | Channel 5 ADC_MUX setting when channel 5 is being measured. |
| 16-14 | R/W | 6 | Channel 4 ADC_MUX setting when channel 4 is being measured. |
| 13-11 | R/W | 0 | Channel 3 ADC_MUX setting when channel 3 is being measured. |
| 10-8 | R/W | 1 | Channel 2 ADC_MUX setting when channel 2 is being measured. |
| 7 | R | 0 | unused |
| 6 | R/W | 0 | VREF_P_MUX setting when channel 2,3..7 is being measured |
| 5 | R/W | 0 | VREF_N_MUX setting when channel 2,3..7 is being measured |
| 4 | R/W | 0 | MODE_SEL setting when channel 2,3..7 is being measured |
| 3 | R/W | 1 | YP_DRIVE_SW setting when channel 2,3..7 is being measured |
| 2 | R/W | 1 | XP_DRIVE_SW setting when channel 2,3..7 is being measured |
| 1 | R/W | 0 | YM_DRIVE_SW setting when channel 2,3..7 is being measured |
| 0 | R/W | 0 | YM_DRIVE_SW setting when channel 2,3..7 is being measured |

SAR_ADC_CHAN_10_SW

Channels 0 and 1 have independent programmable switch settings when either/both of these channels are being measured.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-26 | R | 0 | unused |
| 25-23 | R/W | 2 | Channel 1 ADC MUX setting |
| 22 | R/W | 0 | Channel 1 VREF_P_MUX |
| 21 | R/W | 0 | Channel 1 VREF_N_MUX |
| 20 | R/W | 0 | Channel 1 MODE_SEL |
| 19 | R/W | 1 | Channel 1 YP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 18 | R/W | 1 | Channel 1 XP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 17 | R/W | 0 | Channel 1 YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |
| 16 | R/W | 0 | Channel 1 YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |
| 15-10 | R |  | unused |
| 9-7 | R/W | 3 | Channel 0 ADC MUX setting |
| 6 | R/W | 0 | Channel 0 VREF_P_MUX |
| 5 | R/W | 0 | Channel 0 VREF_N_MUX |
| 4 | R/W | 0 | Channel 0 MODE_SEL |
| 3 | R/W | 1 | Channel 0 YP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 2 | R/W | 1 | Channel 0 XP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 1 | R/W | 0 | Channel 0 YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |
| 0 | R/W | 0 | Channel 0 YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |

SAR_ADC_DETECT_IDLE_SW

IDLE MODE:

When nothing is being measured, the switches should be put into a safe state.  This safe state is accomplished using bits [9:0] below.

DETECT MODE:

When bit [26] is set, the input muxes / switches are configured according to the bits below.   Typically the software configures the switches below to correspond to the detect touch mode.  That is, Y- internal MOSFET is closed so that the Y plane of the touch screen is connected to Ground.  Additionally, the DETECT_EN bit (different register) set to 1 so that the 50k resistor to VDD is connected to X+.  In this configuration, the detect comparator connected to the 50k resistor will be weakly pulled up to VDD through the 50k resistor.  If the user touches the screen, the X and Y planes of the touch screen will contact causing the X+ signal to be pulled to ground.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-27 | R | 0 | unused |
| 26 | R/W | 0 | DETECT_SW_EN: If this bit is set, then bits [25:16] below are applied to the analog muxes/switches of the touch panel controller. |
| 25-23 | R/W | 5 | DETECT MODE ADC MUX setting |
| 22 | R/W | 0 | DETECT MODE VREF_P_MUX setting |
| 21 | R/W | 0 | DETECT MODE VREF_N_MUX setting |
| 20 | R/W | 0 | DETECT MODE MODE_SEL setting |
| 19 | R/W | 1 | DETECT MODE YP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 18 | R/W | 1 | DETECT MODE XP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 17 | R/W | 0 | DETECT MODE YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |
| 16 | R/W | 0 | DETECT MODE YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |
| 15-10 | R |  | Unused |
| 9-7 | R/W | 5 | IDLE MODE ADC MUX setting |
| 6 | R/W | 0 | IDLE MODE VREF_P_MUX setting |
| 5 | R/W | 0 | IDLE MODE VREF_N_MUX setting |
| 4 | R/W | 0 | IDLE MODE MODE_SEL setting |
| 3 | R/W | 1 | IDLE MODE YP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 2 | R/W | 1 | IDLE MODE XP_DRIVE_SW setting: 0: TADC_CH6N = 3.3v 1: TADC_CH6N = floating |
| 1 | R/W | 0 | IDLE MODE YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |
| 0 | R/W | 0 | IDLE MODE YM_DRIVE_SW setting: 0: TADC_CH4N = floating 1: TADC_CH4N = GND |

SAR_ADC_DELTA_10

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-28 | R | 0 | unused |
| 27-16 | R/W | 0 | Channel 1 delta value when delta processing for channel 1 is enabled. |
| 15-12 | R | 0 | unused |
| 11-0 | R/W | 0 | Channel 0 delta value when delta processing for channel 0 is enabled. |

SAR_ADC_REG11

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-28 | R | 0 | reserved |
| 27 | R | 0 | pd_irq_in_f |
| 26 | R | 0 | pd_irq_out |
| 25 | R/W | 0 | pd_irq_clr：pulse |
| 24 | R/W | 0 | pd_irq_en |
| 23 | R/W | 0 | reserved |
| 22 | R/W | 0 | fifo_disable: 1=dout don`t put into fifo |
| 21 | R/W | 0 | period_sample_en: 1=period_sample enable, dout register into chnl reg AO_SAR_ADC_CHNL01/23/45/67 |
| 20-19 | R/W | 0 | filter_tick_sel: 2’b00: 1us tick 2’b01: 10us tick 2’b10: 100us tick 2’b11: 1ms tick |
| 18-16 | R/W | 0 | filter_cnt_sel： 3’b000: 0 tick(no filter) 3’b001: 1x3 tick 3’b010: 2x3 tick ......... |
| 15 | R/W | 0 | reserved |
| 14-9 | R/W | 0 | SARADC_RSV2 |
| 8 | R/W | 0 | SARADC_SAMPLE_SEL |
| 7 | R/W | 0 | SARADC_MODEL_SEL |
| 6 | R/W | 0 | SARADC_IBSEL_COMP |
| 5-0 | R/W | 0 | SARADC_RSV |

SAR_ADC_REG12

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-0 | R/W | 0 | reserved |

SAR_ADC_REG13

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-8 | R | 0 | unused |
| 15-8 | R/W | 0 | SARADC_RSV2 |
| 7-0 | R/W | 0 | reserved |

SAR_ADC_CHNL01

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | dout_chnl1_valid |
| 30-28 | R/W | 0 | dout_chnl1_id |
| 27-16 | R/W | 0 | dout_chnl1 |
| 15 | R/W | 0 | dout_chnl0_valid |
| 14-12 | R/W | 0 | dout_chnl0_id |
| 11-0 | R/W | 0 | dout_chnl0 |

SAR_ADC_CHNL23

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | dout_chnl3_valid |
| 30-28 | R/W | 0 | dout_chnl3_id |
| 27-16 | R/W | 0 | dout_chnl3 |
| 15 | R/W | 0 | dout_chnl2_valid |
| 14-12 | R/W | 0 | dout_chnl2_id |
| 11-0 | R/W | 0 | dout_chnl2 |

SAR_ADC_CHNL45

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | dout_chnl5_valid |
| 30-28 | R/W | 0 | dout_chnl5_id |
| 27-16 | R/W | 0 | dout_chnl5 |
| 15 | R/W | 0 | dout_chnl4_valid |
| 14-12 | R/W | 0 | dout_chnl4_id |
| 11-0 | R/W | 0 | dout_chnl4 |

SAR_ADC_CHNL67

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | dout_chnl7_valid |
| 30-28 | R/W | 0 | dout_chnl7_id |
| 27-16 | R/W | 0 | dout_chnl7 |
| 15 | R/W | 0 | dout_chnl6_valid |
| 14-12 | R/W | 0 | dout_chnl6_id |
| 11-0 | R/W | 0 | dout_chnl6 |

SAR_ADC_TODEMODE_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Sar_adc_cut_enable: 11bit sar_adcout -> 10bit |
| 30 | R/W | 0 | Sar_adc_cut_enable for sar_adc |
| 29 | R/W | 0 | Sar_adc_cut_enable for sar_adc latch data |
| 22:12 | R/W | ‘d1738 | Y2: |
| 10:0 | R/W | ‘d311 | Y1 |

SAR_ADC_TODEMODE_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | ‘d734 | Saradc_coef: (1023<<10)/(y2-y1) |

SAR_ADC_RDY:

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-1 | R/W | 0 | reserved |
| 0 | R | 0 | saradc_rdy: read this bit to own sar adc to normal mode |
|  | W | 0 | saradc_rdy: write 1 to release sar adc to rng mode: when saradc_rdy is 1’b1, when rng_req becomes 1’b1, sar adc works at rng mode |

- TEMP_SENSOR(TOP)

MS1 integrates 1 Temp Sensor at SoC Top.

- Capture temperature by programable tick;

- Store 16 temperature value;

- Can reset all chip if detect high temperature;

- Can generate IRQ by 8 threshold;

TS_CFG_REG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | fall_th3_irq_en |
| 30 | R/W | 0 | fall_th2_irq_en |
| 29 | R/W | 0 | fall_th1_irq_en |
| 28 | R/W | 0 | fall_th0_irq_en |
| 27 | R/W | 0 | rise_th3_irq_en |
| 26 | R/W | 0 | rise_th2_irq_en |
| 25 | R/W | 0 | rise_th1_irq_en |
| 24 | R/W | 0 | rise_th0_irq_en |
| 23 | R/W | 0 | fall_th3_irq_stat_clr |
| 22 | R/W | 0 | fall_th2_irq_stat_clr |
| 21 | R/W | 0 | fall_th1_irq_stat_clr |
| 20 | R/W | 0 | fall_th0_irq_stat_clr |
| 19 | R/W | 0 | rise_th3_irq_stat_clr |
| 18 | R/W | 0 | rise_th2_irq_stat_clr |
| 17 | R/W | 0 | rise_th1_irq_stat_clr |
| 16 | R/W | 0 | rise_th0_irq_stat_clr |
| 15 | R/W | 0 | 1: enable IRQ related function. |
| 14 | R/W | 0 | fast_mode: 0 : downsample unit = 1ms; 1: downsample unit = 1us; |
| 13 | R/W | 0 | clr_hi_temp_stat |
| 12 | R/W | 0 | ts_ana_rset_vbg reset vbg(set 0, if have error place set it plus 01000..) |
| 11 | R/W | 0 | ts_ana_rst_sd reset adc(set 0, if have error place set it plus 01000..) |
| 10 | R/W | 0 | ts_ana_en_vcm enable vcm (disable:0; enable:1) |
| 9 | R/W | 0 | ts_ana_en_vbg enable vbg (disable:0; enable: 1) |
| 8:7 | R/W | 0 | filter hcic mode 0: downsample rate = 128; 1: downsample rate = 256; 2/3: downsample rate = 512; |
| 6 | R/W | 0 | filter ts_out_ctrl; 1: add more delay for filter lock; |
| 5 | R/W | 0 | filter en(disable:0; enable:1) |
| 4 | R/W | 0 | ts_ana_en_iptat, useless. |
| 3 | R/W | 0 | Temp Sensor DEM enable. (disable:0; enable:1) |
| 2:0 | R/W | 0 | Bipolar bias current input control. recommend value : 3. ts_ana_ch_sel; 0: 8’b00000001; 1: 8’b00000011; 2: 8’b00000111; 3: 8’b00001111; 4: 8’b00011111; 5: 8’b00111111; 6: 8’b01111111; 7: 8’b11111111; |

TS_CFG_REG2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | hi_temp_enable |
| 30 | R/W | 0 | reset_en, if = 0, will not reset all chip; |
| 27:16 | R/W | 0 | high temperature times, if continuous detect high temperature, then will reset all chip. |
| 15:0 | R/W | 0 | high temperature threshold, if temperature value > this th , mean detected once high temperature |

TS_CFG_REG3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R/W | 0 |  |
| 15:0 | R/W | 0 | down_sample rate |

TS_CFG_REG4

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 23:12 | R/W | 0 | rise_th0 |
| 11:0 | R/W | 0 | rise_th1 |

TS_CFG_REG5

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 23:12 | R/W | 0 | rise_th2 |
| 11:0 | R/W | 0 | rise_th3 |

TS_CFG_REG6

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 23:12 | R/W | 0 | fall_th0 |
| 11:0 | R/W | 0 | fall_th1 |

TS_CFG_REG7

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 23:12 | R/W | 0 | fall_th2 |
| 11:0 | R/W | 0 | fall_th3 |

TS_STAT0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:18 | R | 0 | detect_hi_temp_cnt |
| 17 | R | 0 | detected_hi_temp_stat |
| 16 | R | 0 | filter lock |
| 15:0 | R | 0 | filter out |

TS_STAT1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 8 | R | 0 | hi_temp_stat |
| 7 | R | 0 | fall_th3_irq |
| 6 | R | 0 | fall_th2_irq |
| 5 | R | 0 | fall_th1_irq |
| 4 | R | 0 | fall_th0_irq |
| 3 | R | 0 | rise_th3_irq |
| 2 | R | 0 | rise_th2_irq |
| 1 | R | 0 | rise_th1_irq |
| 0 | R | 0 | rise_th0_irq |

TS_STAT2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D2 |
| 15:0 | R | 0 | temperature value D1 |

TS_STAT3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D4 |
| 15:0 | R | 0 | temperature value D3 |

TS_STAT4

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D6 |
| 15:0 | R | 0 | temperature value D5 |

TS_STAT5

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D8 |
| 15:0 | R | 0 | temperature value D7 |

TS_STAT6

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D10 |
| 15:0 | R | 0 | temperature value D9 |

TS_STAT7

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D12 |
| 15:0 | R | 0 | temperature value D11 |

TS_STAT8

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D14 |
| 15:0 | R | 0 | temperature value D13 |

TS_STAT9

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | temperature value D16 |
| 15:0 | R | 0 | temperature value D15 |

- debug_i2c

Debug_i2c module is for register w/r like PQ test, SPIFC access to program flash through DDC interface.

Internal HDMI0_I2Cs & debug_i2c0 share HDMIRX DDC 0 PAD.

Internal HDMI1_I2Cs & debug_i2c1 share HDMIRX DDC 1 PAD.

Debug_i2c with Bus Master attribute.

Debug and HDMI I2C has its own slave address(3B for debug_i2c 0, 3C for debug_i2c 1).

Host PC use slave address to select HDMIRX I2C or debug i2c.

I2C_DBG_T_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:13 | R/W | 0x0 | Reserved |
| 12:10 | R/W | 0x0 | debug_i2c 1 cntl_sample_dly |
| 9:8 | R/W | 0x1 | debug_i2c 1 cntl_filter_sel |
| 7:5 | R/W | 0x0 | Reserved |
| 4:2 | R/W | 0x1 | debug_i2c 0 cntl_sample_dly |
| 1:0 | R/W | 0x0 | debug_i2c 0 cntl_filter_sel |

I2C_DBG_PWD

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0x0 | debug_i2c password |

- spifc

PI_FLASH_CMD

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | READ command. 1 = read. When it becames 0, the read command is finished. The READ command could be (0xEB, 0x6B, 0xBB, 0x3B, 0x0B, 0x03). By default is 0x0B. One read command will read erilog 32x8bits data. And saved in data cache. |
| 30 | R/W | 0 | WREN command. (0x06) |
| 29 | R/W | 0 | WRDI command. ( 0x04). |
| 28 | R/W | 0 | RDID command. (0x9f). |
| 27 | R/W | 0 | RDSR command. (0x05). |
| 26 | R/W | 0 | WRSR command. (0x01). |
| 25 | R/W | 0 | Page program command. (0xAD or 0x02). |
| 24 | R/W | 0 | SE command (0x20). |
| 23 | R/W | 0 | BE command ( 0xD8). |
| 22 | R/W | 0 | CE command.(0xC7). |
| 21 | R/W | 0 | Deep Power Down command(0xB9). |
| 20 | R/W | 0 | RES command. (0xAB). |
| 19 | R/W | 0 | HPM command.(0xA3). (Just For winbond SPI flash). |
| 18 | R/W | 0 | USER defined command. |
| 17:0 | R/W | 0 | Reserved for future. |

SPI_FLASH_ADDR

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | The address[31:0] of the user command |

SPI_FLASH_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:27 | R/W | 0 | Reserved. |
| 26 | R/W | 0 | Write bit order. 1 = 0, 1, 2, 3, 4, 5, 6, 7. 0 = 7, 6, 5, 4, 3, 2, 1, 0. |
| 25 | R/W | 0 | Read bit order. . 1 = 0, 1, 2, 3, 4, 5, 6, 7. 0 = 7, 6, 5, 4, 3, 2, 1, 0. |
| 24 | R/W | 0 | Fast read QIO mode. |
| 23 | R/W | 0 | Fast read DIO mode. |
| 22 | R/W | 0 | Write 2 bytes status mode. For some of winbond SPI flash, the status register is 16bits. |
| 21 | R/W | 1 | SPI flash WP pin value if use SPI flash WP pin as write protection. |
| 20 | R/W | 0 | Fast read QOUT mode. |
| 19 | R/W | 1 | 1 = SPI share pins with SDRAM. 0 = doesn’t share. |
| 18 | R/W | 0 | SPI hold mode. 1=SPI controller would use SPI hold function. 0 = SPI controller won’t use hold function. The SPI flash hold pin can be tie high on the board. Or SPI controller can use hold pin as QIO/QOUT mode. |
| 17 | R/W | 1 | 1 = enable AHB request. 0 = disable AHB request when you reconfigure SPI controller or running APB bus commands. |
| 16 | R/W | 0 | 1 =enable SST SPI Flash aai command. The APB bus PP command will send AAI command. |
| 15 | R/W | 1 | 1 = release from Deep Power-Down command is with read electronic signature. |
| 14 | R/W | 0 | Fast read DOUT mode. |
| 13 | R/W | 1 | Fast read mode. AHB bus read requirement and APB bus read command use the command 0x0Bh. |
| 12:0 | R/W | 0 | Reserved for future. |

SPI_FLASH_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 5 | SPI Clock cycles for SPI flash timing requirement tCSH. |
| 27:16 | R/W | 0xfff | SPI Clock cycles for SPI flash timing requirement tRES. |
| 15:0 | R/W | 0x0120 | System clock cycles for SPI bus timer. In SPI share bus and SPI hold function mode. SPI bus timer used , if SPI use the bus for a limit time, SPI controller will diassert SPI hold pin to halt the SPI Flash, and give the bus control to SDRAM. |

SPI_FLASH_STATUS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:24 | R/W | 0 | Reserved. |
| 23:16 | R/W | 0 | For winbond SPI flash, this 8 bits used for DIOmode M7~M0, |
| 15:0 | R/W | 0 | SPI status register value. WRSR command will write this value to SPI flash status. RDSR or RES command will save the read result to this register. |

When SPI controller in the slave mode, this register are the status for the SPI master to read out.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | In SPI Slave mode, the read status of the user command |

SPI_FLASH_CTRL2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | Delay cycle number of SPI_CS input in SPI slave mode or SPI_CS output in SPI master mode 0= not delay, 1 = delayed by 1 cycle of system clock, 2 = delayed by 2 cycles of system clock , … |
| 27:26 | R/W | 0 | delay mode of SPI_CS input in SPI slave mode or SPI_CS output in SPI master mode 0= not latched by the edges of SPI_CK 1= latched by the falling edges of SPI_CK 2 = latched by the rising edges of SPI_CK |
| 25:23 | R/W | 0 | Delay cycle number of SPI Data from SPI Master to SPI Slave In SPI master mode, it is for data outputs; in SPI slave mode, it is for data inputs. 0= not delay, 1 = delayed by 1 cycle of system clock, 2 = delayed by 2 cycles of system clock , … |
| 22:21 | R/W | 0 | Delay mode of SPI Data from SPI Master to SPI Slave In SPI master mode, it is for data outputs; in SPI slave mode, it is for data inputs. 0= not latched by the edges of SPI_CK 1= latched by the falling edges of SPI_CK 2 = latched by the rising edges of SPI_CK |
| 20:18 | R/W | 0 | Delay cycle number of SPI Data from SPI Slave to SPI Master. In SPI master mode, it is for data inputs; in SPI slave mode, it is for data outputs. 0= not delay, 1 = delayed by 1 cycle of system clock, 2 = delayed by 2 cycles of system clock , … |
| 17:16 | R/W | 0 | Delay mode of SPI Data from SPI Slave to SPI Master. In SPI master mode, it is for data inputs; in SPI slave mode, it is for data outputs. 0= not latched by the edges of SPI_CK 1= latched by the falling edges of SPI_CK 2 = latched by the rising edges of SPI_CK |
| 15:12 | R/W | 0 | In SPI master mode, SPI_CK rising edge mode 4’b1000 = later by 1/4 cycle of SPI_CK 4’b1001 = later by 1/8 cycle of SPI_CK 4’b1010 = later by 1/16 cycle of SPI_CK 4’b1011 = later by 1/32 cycle of SPI_CK 4’b1100 = earlier by 1/4 cycle of SPI_CK 4’b1101 = earlier by 1/8 cycle of SPI_CK 4’b1110 = earlier by 1/16 cycle of SPI_CK 4’b1111 = earlier by 1/32 cycle of SPI_CK Others = Normal |
| 11:8 | R/W | 0 | In SPI master mode, SPI_CK falling edge mode 4’b1000 = later by 1/4 cycle of SPI_CK 4’b1001 = later by 1/8 cycle of SPI_CK 4’b1010 = later by 1/16 cycle of SPI_CK 4’b1011 = later by 1/32 cycle of SPI_CK 4’b1100 = earlier by 1/4 cycle of SPI_CK 4’b1101 = earlier by 1/8 cycle of SPI_CK 4’b1110 = earlier by 1/16 cycle of SPI_CK 4’b1111 = earlier by 1/32 cycle of SPI_CK Others = Normal |
| 7:4 | R/W | 1 | In master mode, SPI clock cycles for SPI hold timing. |
| 3:0 | R/W | 1 | In master mode, SPI clock sycles for SPI setup timing. SPI setup time and SPI hold time is used to configure how soon the controller can enable spi_cs_n after the controller get the bus and how long the controller still keep the bus after the spi_cs_n become to be high. |

SPI_FLASH_CLOCK

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 1 | 1=SPI clock frequency is same as system clock. 0 = SPI clock frequency will use clock divider. |
| 30:18 | R/W | 0 | Clock counter for Pre-scale divider: 0= not pre-scale divider, 1= pre-scale divided by 2, 2= pre-scale divided by 3, …., |
| 17:12 | R/W | 0 | Clock counter for clock divider. |
| 11:6 | R/W | 0 | Clock high counter in SPI master mode. In SPI slave mode, it is for the delay counter for the rising edges of spi_ck_i |
| 5:0 | R/W | 0 | Clock low counter, in SPI master mode. In SPI slave mode, it is for the delay counter for the falling edges of spi_ck_i If the SPI clock frequcncy = sys_clock_frequency / n. Then the clock divider counter = n – 1; the clock high counter = n /2 – 1; the clock low counter = n– 1; For example, if you want to SPI clock erilog is divided by 2 of the system clock. The clock divider counter = 1, clock high counter = 0, clock low counter = 1. For SPI clock frequency = system clock / 4. The clock divider counter = 3, clock high counter = 1, clock low counter = 3. |

SPI_FLASH_USER

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 1 | USER command COMMAND bit. 1 = user command includes command. 0 = no command. If some SPI slaves may support 2/4 IO at the first cycle, clear this bit. |
| 30 | R/W | 0 | USER command ADDRESS bit. 1 = user command includes address. 0 = no address. |
| 29 | R/W | 0 | USER command DUMMY bit. 1= user command includes Dummy bytes. |
| 28 | R/W | 0 | USER command DIN bit. 1 = user command includes data in. 0 = no data in. |
| 27 | R/W | 0 | USER command DO bit. 1 = user command includes data output. 0 = no data output. If both DIN and DO are valid, SPI master is firstly in data output state and then in data input state. If all of DUMMY, DO and DIN are valid, SPI master is firstly in data output state and then in dummy state, finally in data input state. |
| 26 | R/W | 0 | USER command dummy idle bit. 1= no SPI clock toggling in dummy state. 0= normal |
| 25 | R/W | 0 | USER command highpart bit for SPI_DOUT stage. It is for data-output in spi master mode and for data-input in spi slave mode. 1 = only high half part of buffer are used. 0 = low half part or the whole 64bytes are used. |
| 24 | R/W | 0 | USER command highpart bit for SPI_DIN stage. It is for data-input in spi master mode and for data-output in spi slave mode. 1 = only high half part of buffer are used. 0 = low half part or the whole 64bytes are used. |
| 23 | R/W | 0 | User command external hold bit for prep. 1 = in prep state, SPI master controller can be hold by the external pin SPI_HOLD |
| 22 | R/W | 0 | User command external hold bit for command. 1 = in command state, SPI master controller can be hold by the external pin SPI_HOLD |
| 21 | R/W | 0 | User command external hold bit for address. 1 = in address state, SPI master controller can be hold by the external pin SPI_HOLD |
| 20 | R/W | 0 | User command external hold bit for dummy. 1 = in dummy state, SPI master controller can be hold by the external pin SPI_HOLD |
| 19 | R/W | 0 | User command external hold bit for data input. 1 = in data input state, SPI master controller can be hold by the external pin SPI_HOLD |
| 18 | R/W | 0 | User command external hold bit for data output. 1 = in data output state, SPI master controller can be hold by the external pin SPI_HOLD |
| 17 | R/W | 1 | User command external hold polarity bit. 1 = high is valid for hold, 0 = low is valid for hold. |
| 16 | R/W | 0 | Single DIO mode: Data output and input apply only 1 wire. |
| 15 | R/W | 0 | Fast write QIO mode. |
| 14 | R/W | 0 | Fast write DIO mode. |
| 13 | R/W | 0 | Fast write QOUT mode. |
| 12 | R/W | 0 | Fast write DOUT mode. |
| 11 | R/W | 0 | Write byte order. 0 = d[7:0], d[15:8], d[23:16], d[31:24]. 1 = d[31:24], d[23:16], d[15:8], d[7:0] |
| 10 | R/W | 0 | Read byte order. . 0 = d[7:0], d[15:8], d[23:16], d[31:24]. 1 = d[31:24], d[23:16], d[15:8], d[7:0] |
| 9:8 | R/W | 0 | AHB endian mode: 0= little-endian; 1= big-endian; 2~3 reserved |
| 7 | R/W | 0 | In SPI master mode, the clock output edge bit: 0 = SPI_CK is inverted, 1 = SPI_CK is not inverted |
| 6 | R/W | 1 | In SPI slave mode, the clock input edge bit: 0 = SPI_CK_I is inverted, 1 = SPI_CK_I is not inverted |
| 5 | R/W | 0 | SPI CS setup bit: 1 = valid in prep state |
| 4 | R/W | 0 | SPI CS hold bit: 1 = valid in done state |
| 3 | R/W | 0 | AHB-read apply the configurations of user-command, such as command value, bit-length,… |
| 2 | R/W | 1 | Backward Compatible: 1 = compatible to Apollo SPI This bit affect the three registers: “SPI Flash Commmand Register”, “SPI Address Register” and “SPI Control Register” |
| 1 | R/W | 0 | AHB-read support 4byte address, when AHB-read apply the configurations of user-command. 1 = 4byte address, 0 = 3byte address |
| 0 | R/W | 0 | In SPI master mode, Enable bit for Data input during SPI_DOUT stage. 1 = enable; 0 = disable This bit shall not be used in 2/4wire or SIO. When this bit is 1, during SPI_DOUT stage, data input are stored into cacheline/buffer from address 0, i.e., Bit24 is not controlling the start address. The data output can be specified by bit25 |

SPI_FLASH_USER1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:26 | R/W | 0 | USER command bit number for address state 0 = 1 bit, 1= 2 bits, … |
| 25:17 | R/W | 0 | USER command bit number for data output state 0 = 1 bit, 1= 2 bits, … |
| 16:8 | R/W | 0 | USER command bit number for data input state 0 = 1 bit, 1= 2 bits, … |
| 7:0 | R/W | 0 | USER command cycle number for dummy state 0 = 1 cycle, 1= 2 cycles, … |

SPI_FLASH_USER2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | USER command bit number for command state 0 = 1 bit, 1= 2 bits, … |
| 27:16 | R/W | 0 | Reserved |
| 15:0 | R/W | 0 | The command content of the user command |

SPI_FLASH_USER3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | In SPI Master mode, the address[63:32] of the user command In SPI Slave mode, the write status of the user command |

SPI_FLASH_USER4

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Pin swap when it is in DIN stage and SPI data input are 4wire. This feature is for Ubec Zigbee chips. 1 = swap between {spi_q, spi_d_i} and {spi_hold_i, spi_wp_i} 0 = normal |
| 30 | R/W | 0 | In SPI Master mode, CS keep active after a transition. 1 = enable; 0 = disable |
| 29 | R/W | 0 | Idle edge of SPI_CK 0 = low when it is idle 1 = high when it is idle |
| 28:24 | R/W | 0 | Reserved |
| 23 | R/W | 0 | In the SPI slave mode, spi_cs_i polarity: 1= high voltage is active 0= low voltage is active |
| 22:21 | R/W | 0 | In the SPI slave mode, spi_ck_i and spi_cs_i source pins 0=SPI_CK and SPI_CS pins, respectively 1=SPI_CS2 and SPI_CS1 pins, respectively 2=SPI_HOLD and SPI_WP pins, respectively |
| 20 | R/W | 0 | SPI_CS2 and SPI_CS1 pin function MUX 0= spi_ck and spi_cs in the SPI master mode 1= input pads for spi_ck_i and spi_cs_i, respectively, in the SPI slave mode |
| 19 | R/W | 0 | SPI_CK and SPI_CS pin function MUX 0= spi_ck and spi_cs in the SPI master mode 1= input pads for spi_ck_i and spi_cs_i, respectively, in the SPI slave mode |
| 18:17 | R/W | 0 | SPI_HOLD and SPI_WP pin function MUX 0= normal 1= spi_q and spi_d, respectively 2= spi_cs3 and spi_cs2, repectively 3= input pads for spi_ck_i and spi_cs_i, respectively, in the SPI slave mode |
| 16 | R/W | 0 | SPI_D and SPI_Q switch 1= SPI_D and SPI_Q pin-functions are swapped 0= normal |
| 15:11 | R/W | 0 | In Master mode, these are spi_ck MUX bit[4:0] for spi_cs4 (SPI_HOLD pin), spi_cs3 (SPI_WP pin), spi_cs2, spi_cs1 and spi_cs, respectively 1= this pin is spi_ck, if this pin is not idle 0= this pin is spi_cs, if this pin is not idle |
| 10:6 | R/W | 0 | In Master mode, these are polarity bit[4:0] for spi_cs4 (SPI_HOLD pin), spi_cs3 (SPI_WP pin), spi_cs2, spi_cs1 and spi_cs, respectively 1= high voltage is active 0= low voltage is active |
| 5:0 | R/W | 0x1E | In Master mode, these are idle bit[5:0] for SPI_CK, for spi_cs4 (SPI_HOLD pin), spi_cs3 (SPI_WP pin), spi_cs2, spi_cs1 and spi_cs, respectively 1= idle, i.e., the spi_ck signal is 0 or the spi_cs is at the inactive level 0= active if SPI controller is working |

SPI_FLASH_SLAVE

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | SPI controller SW reset: 1 = reset, 0 = none |
| 30 | R/W | 0 | SPI slave mode: 1 = slave, 0 = master |
| 29 | R/W | 0 | In SPI slave mode, the enable bit for the command of write-buffer-and-read-buffer 0= disable, 1=enable |
| 28 | R/W | 0 | In SPI slave mode, the enable bit for the command of write-status-and-read-status 0= disable, 1=enable |
| 27 | R/W | 0 | SPI slave command define enable 0=Apply the last 3bits of Flash commands and two extra command, please refer to In SPI slave mode, SPI-Flash-Lite Commands are supported. 1=Apply the user defined command in SPI Slave register 3 |
| 26:4 | R/W | 0 | Reserved |
| 11:10 | R/W | 0 | spi_cs_i recovery mode: 0= and 1= or 2= normal 3= delayed |
| 9:5 | R/W | 0 | Interrupt enable for bit4:0 1= enable, 0=disable |
| 4 | R/W | 0 | A SPI transition is done. (whatever it is in SPI master mode or SPI slave mode) |
| 3 | R/W | 0 | In SPI slave mode, a status write is done |
| 2 | R/W | 0 | In SPI slave mode, a status read is done |
| 1 | R/W | 0 | In SPI slave mode, a buffer write is done |
| 0 | R/W | 0 | In SPI slave mode, a buffer read is done |

SPI_FLASH_SLAVE1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:27 | R/W | 0 | In SPI slave mode, status bit number 0 = 1 bit, 1= 2 bits, … |
| 26 | R/W | 0 | In SPI slave mode, status fast read/write enable bit: 1 = enable, 0 = disable |
| 25 | R/W | 0 | In SPI slave mode, status read back enable bit: 1 = reading status is written status in SPI User register 3, 0 = reading status is in SPI Status register. |
| 24:16 | R/W | 0 | In SPI slave mode, buffer bit number 0 = 1 bit, 1= 2 bits, … |
| 15:10 | R/W | 0 | In SPI slave mode, address bit number for reading buffer 0 = 1 bit, 1= 2 bits, … |
| 9:4 | R/W | 0 | In SPI slave mode, address bit number for writing buffer 0 = 1 bit, 1= 2 bits, … |
| 3 | R/W | 0 | In SPI slave mode, dummy enable bit for writing status 1=enable, 0=disable |
| 2 | R/W | 0 | In SPI slave mode, Dummy enable bit for reading status 1=enable, 0=disable |
| 1 | R/W | 0 | In SPI slave mode, Dummy enable bit for writing buffer 1=enable, 0=disable |
| 0 | R/W | 0 | In SPI slave mode, Dummy enable bit for reading buffer 1=enable, 0=disable |

SPI_FLASH_SLAVE2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:24 | R/W | 0 | In SPI slave mode, Dummy cycle number for writing buffer 0 = 1 cycle, 1= 2 cycles, … |
| 23:16 | R/W | 0 | In SPI slave mode, Dummy cycle number for reading buffer 0 = 1 cycle, 1= 2 cycles, … |
| 15:8 | R/W | 0 | In SPI slave mode, Dummy cycle number for writing status 0 = 1 cycle, 1= 2 cycles, … |
| 7:0 | R/W | 0 | In SPI slave mode, Dummy cycle number for reading status 0 = 1 cycle, 1= 2 cycles, … |

SPI_FLASH_SLAVE3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:24 | R/W | 0 | In SPI slave mode, Command value for writing status, when bit27 “SPI slave command define enable” in SPI Slave register is 1 |
| 23:16 | R/W | 0 | In SPI slave mode, Command value for reading status, when bit27 “SPI slave command define enable” in SPI Slave register is 1 |
| 15:8 | R/W | 0 | In SPI slave mode, Command value for writing buffer, when bit27 “SPI slave command define enable” in SPI Slave register is 1 |
| 7:0 | R/W | 0 | In SPI slave mode, Command value for reading buffer, when bit27 “SPI slave command define enable” in SPI Slave register is 1 |

SPI_FLASH_C0~7

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | Cache line Word 0~7. Cache is used to read data both for AHB or APB read command. Cache is also used for APB page programming etc. |

SPI_FLASH_B8~15

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | Buffer Word 8. Buffer is used to read/write data only for APB read/write user commands. |