MS1 System Register Specification

| Date | Version | Author | Change |
| --- | --- | --- | --- |
| 2025/7/1 | v0.1 | liang.zhao | Copy from t6X and update for MS1 |
| 2025/7/16 | v0.2 | tao.hu | update for irq_ctrl & efuse_top |
| 2025/7/16 | v0.3 | xiaoyong.ma | update ana_ctrl/pwr_ctrl/clk_ctrl |
| 2025/7/17 | v0.4 | faying.luo | update ao_reset_ctrl/ao_sys_ctrl |
| 2025/7/18 | v0.5 | feng.hu | update ao_pad_ctrl /ee_pad_ctrl /edp_lvds_phy_dig_top |
| 2025/8/26 | v0.6 | liang.zhao | update some missed register |
| 2025/8/26 | v0.7 | liang.zhao | update some missed register |
| 2025/8/28 | v0.8 | guojin.zhang | update some register |
| 2025/9/1 | v0.9 | liang.zhao | add aocpu register |

Table of Contents

Table of Contents

Contents

1. Module address list:	10

2. AO_RESET_CTRL	11

RESETCTRL_RESET0~1	12

RESETTRL_RESET0~1_LEVEL	12

RESETCTRL_RESET0~1_MASK	12

RESETCTRL_RESET_HOLD_CYC	12

RESETCTRL_WATCHDOG_CTRL0	13

RESETCTRL_WATCHDOG_CTRL1	13

RESETCTRL_WATCHDOG_CNT	13

RESETCTRL_WATCHDOG_CLR	14

RESETCTRL_WATCHDOG_IRQ_OFFSET	14

3. AO_PAD_CTRL	15

AO_PADCTRL_PIN_MUX_REG0	17

AO_PADCTRL_PIN_MUX_REG1	17

AO_PADCTRL_PIN_MUX_REG2	17

AO_PADCTRL_PIN_MUX_REG3	18

AO_PADCTRL_GPIO_IRQ_CTRL0	18

AO_PADCTRL_GPIO_IRQ_CTRL1	18

AO_PADCTRL_GPIO_IRQ_CTRL2	20

AO_PADCTRL_GPIO_MSR_CTRL0	20

AO_PADCTRL_GPIOAO_I	21

AO_PADCTRL_GPIOAO_O	21

AO_PADCTRL_GPIOAO_OEN	21

AO_PADCTRL_GPIOAO_IEN	21

AO_PADCTRL_GPIOAO_PULL_UP	21

AO_PADCTRL_GPIOAO_PULL_DOWN	21

AO_PADCTRL_GPIOAO_DS	21

AO_PADCTRL_GPIOW_I	22

AO_PADCTRL_GPIOW_O	22

AO_PADCTRL_GPIOW_OEN	22

AO_PADCTRL_GPIOW_IEN	22

AO_PADCTRL_GPIOW_PULL_UP	22

AO_PADCTRL_GPIOW_PULL_DOWN	22

AO_PADCTRL_GPIOW_DS	22

AO_PADCTRL_TESTN_I	23

AO_PADCTRL_TESTN_O	23

AO_PADCTRL_TESTN_OEN	23

AO_PADCTRL_TESTN_IEN	23

AO_PADCTRL_TESTN_PULL_UP	23

AO_PADCTRL_TESTN_PULL_DOWN	23

AO_PADCTRL_TESTN_DS	23

AO_PADCTRL_UART_DISABLE_CTRL0	24

AO_PADCTRL_ANALOG_I	25

AO_PADCTRL_ANALOG_EN	25

AO_PADCTRL_GPIOW_CTRL0	25

AO_PADCTRL_GPIOW_CTRL1	26

4. EE_PAD_CTRL	27

EE_PADCTRL_PIN_MUX_REG0	27

EE_PADCTRL_PIN_MUX_REG1	28

EE_PADCTRL_PIN_MUX_REG2	28

EE_PADCTRL_GPIO_IRQ_CTRL0	29

EE_PADCTRL_GPIO_IRQ_CTRL1	29

EE_PADCTRL_GPIO_IRQ_CTRL2	30

EE_PADCTRL_GPIO_MSR_CTRL0	31

EE_PADCTRL_GPIOH_I	31

EE_PADCTRL_GPIOH_O	31

EE_PADCTRL_GPIOH_OEN	31

EE_PADCTRL_GPIOH_IEN	32

EE_PADCTRL_GPIOH_PULL_UP	32

EE_PADCTRL_GPIOH_PULL_DOWN	32

EE_PADCTRL_GPIOH_DS	32

EE_PADCTRL_UART_DISABLE_CTRL0	32

EE_PADCTRL_DEBUG_SEL	32

5. AO_CLK_CTRL	34

AO_CLKCTRL_OSCIN_CTRL	34

AO_CLKCTRL_RTC_BY_OSCIN_CTRL0	35

AO_CLKCTRL_RTC_BY_OSCIN_CTRL1	35

AO_CLKCTRL_RTC_CTRL	36

AO_CLKCTRL_MBIST_ATSPEED_CTRL	36

AO_CLKCTRL_SYS_CLK_CTRL0	36

AO_CLKCTRL_SYS_CLK_EN0_REG0 & AO_CLKCTRL_CLK81_GATE_CTRL	38

AO_CLKCTRL_AHB_CLK_CTRL0	38

AO_CLKCTRL_TST_CTRL0/1	39

AO_CLKCTRL_CECB_CLK_CTRL0	40

AO_CLKCTRL_CECB_CLK_CTRL1	42

AO_CLKCTRL_CLK12_24_CTRL	42

AO_CLKCTRL_PWM_CLK_(AD_)**_CTRL	43

AO_CLKCTRL_GEN_CLK_CTRL	44

AO_CLKCTRL_TIMEBASE_CTRL0	45

AO_CLKCTRL_TIMEBASE_CTRL1	46

AO_CLKCTRL_HRX_CLK_CTRL0	46

AO_CLKCTRL_HRX_CLK_CTRL1	47

AO_FIX_PLL_CTRL0	47

AO_FIX_PLL_CTRL1	47

AO_FIX_PLL_CTRL2	47

AO_FIX_PLL_STS	47

6. EE_CLK_CTRL	48

EE_CLKCTRL_TST_CTRL0/1	48

EE_CLKCTRL_VP_OPT_CLK_CTRL	49

EE_CLKCTRL_BCON_CLK_CNTL	50

EE_CLKCTRL_TX_DET_CLK_CTRL	50

EE_CLKCTRL_APB2_CLK_CTRL	51

EE_CLKCTRL_SPISG_CLK_CTRL	51

EE_CLKCTRL_PWM_CLK_(EH_)**_CTRL	52

EE_CLKCTRL_SAR_CLK_CTRL0	53

EE_CLKCTRL_TS_CLK_CTRL	53

EE_CLKCTRL_AXI_CLK_CTRL0	54

EE_CLKCTRL_CLK81_GATE_CTRL & EE_CLKCTRL_AHB_CLK_GATE_CTRL	55

EE_CLKCTRL_MBIST_ATSPEED_CTRL	55

EE_CLKCTRL_GEN_CLK_CTRL	55

EE_CLKCTRL_HRX_CLK_CTRL1	57

EE_CLKCTRL_HRX_CLK_CTRL2	57

EE_CLKCTRL_VDOUT_CLK_CTRL	58

7. AO_SYS_CTRL	59

SYSCTRL_METAL_REV0	59

SYSCTRL_METAL_REV1	59

SYSCTRL_METAL_REV2	59

SYSCTRL_CHIP_ID	59

SYSCTRL_WRITEONCE_REG0	59

SYSCTRL_WRITEONCE_REG1	59

SYSCTRL_WRITEONCE1_REG0	59

SYSCTRL_WRITEONCE_STICKY_REG0	60

SYSCTRL_WRITEONCE_STICKY_REG1	60

SYSCTRL_TIMERA_CTRL	60

SYSCTRL_TIMERA	61

SYSCTRL_TIMERE/F_CTRL	61

SYSCTRL_TIMERE/F	62

SYSCTRL_TIMERE/F_HI	62

SYSYSCTRL_ALERT_STS_STICKY	63

SYSCTRL_ALERT_ENABLE	63

SYSYSCTRL_DEBUG_REG0~4	63

SYSCTRL_STATUS_REG0~3	63

SYSCTRL_STICKY_REG0~3	63

SYSCTRL_CEC_STICKY_REG0~3	63

SYSCTRL_MSG_INDEX0~1_STICKY	63

SYSYSCTRL_BUS_CFG0	64

SYSCTRL_HDMIRXPLL_TEST_CTRL	64

SYSCTRL_SCAN_PW0	65

SYSCTRL_SCAN_PW1	65

SYSCTRL_SCAN_PW2	65

SYSCTRL_SCAN_PW3	65

SYSCTRL_TEST_MUX_SEL	65

SYSCTRL_TEST_MUX0/1/2/3/4/5	66

SYSCTRL_EFUSE_INFO0	66

SYSCTRL_EFUSE_INFO1	66

SYSCTRL_EFUSE_INFO2	66

SYSCTRL_LDO_CTRL	66

8. EE_SYS_CTRL	67

EE_SYSCTRL_BUS_CFG0	67

EE_SYSCTRL_BUS_CFG1	67

EE_SYSCTRL_NIC_CFG0	67

EE_SYSCTRL_VGD_CTRL	68

EE_SYSCTRL_SPISG_TRIG0	68

EE_SYSCTRL_OSC_RING_CTRL0	69

EE_SYSCTRL_OSC_RING_CTRL1	69

9. IRQ_CTRL	70

IIRQCTRL_IRQ_MODE0~2	71

IRQCTRL_IRQ_LATCH0~2	71

IRQCTRL_IRQ_LATCH_CLR0~2	71

IRQCTRL_IRQ_MASK0~2_AOCPU	71

RQCTRL_IRQ_INV0~2	71

IRQCTRL_SOFT_IRQ	71

10. Efuse_top	72

11. aocpu	73

AOCPU_CPU_VECTOR	73

AOCPU_CPU_CTRL0	73

AOCPU_CPU_CTRL1	73

AOCPU_STS00_BYCPU0~7	73

AOCPU_IRQ_TYPE0/1/2	74

AOCPU_TIMER_DIV	74

AOCPU_TIMER_DBG_DIV	74

12. PWR_CTRL	75

PWRCTRL_PWR_ACK0	75

PWRCTRL_ISO_EN0	75

PWRCTRL_PWR_OFF0	76

PWRCTRL_FOCRST0	76

PWRCTRL_MEM_PD0	76

PWRCTRL_MEM_PD1	76

PWRCTRL_MEM_PD2	77

PWRCTRL_MEM_PD3	77

PWRCTRL_MEM_PD4	77

13. ANA_CTRL	78

ANACTRL_HIFI_PLL_CTRL0	78

ANACTRL_HIFI_PLL_CTRL1	78

ANACTRL_HIFI_PLL_CTRL2	78

ANACTRL_HIFI_PLL_CTRL3	79

ANACTRL_HIFI_PLL_STS	80

ANACTRL_DISP_PLL_CTRL0	80

ANACTRL_DISP_PLL_CTRL1	80

ANACTRL_DISP_PLL_CTRL2	81

ANACTRL_DISP_PLL_CTRL3	81

ANACTRL_DISP_PLL_STS	82

14. edp_lvds_phy_dig_top	83

ANACTRL_TCON_PLL0_CNTL0	83

ANACTRL_TCON_PLL0_CNTL1	83

ANACTRL_TCON_PLL0_CNTL2	84

ANACTRL_TCON_PLL0_CNTL3	85

ANACTRL_TCON_PLL0_STS	85

ANACTRL_DIF_PHY_CNTL1	86

ANACTRL_DIF_PHY_CNTL2	86

ANACTRL_DIF_PHY_CNTL3	86

ANACTRL_DIF_PHY_CNTL4	86

ANACTRL_DIF_PHY_CNTL5	86

ANACTRL_DIF_PHY_CNTL6	86

ANACTRL_DIF_PHY_CNTL7	87

ANACTRL_DIF_PHY_CNTL8	87

ANACTRL_DIF_PHY_CNTL9	87

ANACTRL_DIF_PHY_CNTL10	87

ANACTRL_DIF_PHY_CNTL11	87

ANACTRL_DIF_PHY_CNTL12	88

ANACTRL_DIF_PHY_CNTL13	88

ANACTRL_DIF_PHY_CNTL14	88

ANACTRL_LVDS_TX_PHY_CNTL1	88

ANACTRL_VID_PLL_CLK_DIV	89

ANACTRL_TCON_PLL_VLOCK	89

ANACTRL_TCON_PLL0_STS	90

- Module address list:

| module name | size(KB) | start | end |
| --- | --- | --- | --- |
| ao_clk_ctrl | 8 | FE000000 | FE001FFF |
| ao_reset_ctrl | 8 | FE002000 | FE003FFF |
| ao_pad_ctrl | 8 | FE004000 | FE005FFF |
| irq_ctrl | 8 | FE00A000 | FE00BFFF |
| pwr_ctrl | 8 | FE00C000 | FE00DFFF |
| ao_sys_ctrl | 8 | FE010000 | FE011FFF |
| debug_i2c | 8 | FE016000 | FE017FFF |
| tempsensor | 8 | FE01C000 | FE01DFFF |
| efuse_top | 8 | FE040000 | FE041FFF |
| ao_cpu | 8 | FE09C000 | FE09DFFF |
| ee_clk_ctrl | 8 | FE300000 | FE301FFF |
| ee_pad_ctrl | 8 | FE302000 | FE303FFF |
| ee_ana_ctrl | 8 | FE304000 | FE305FFF |
| msr_clk | 8 | FE308000 | FE309FFF |
| edp_lvds_phy_dig_top | 8 | FE30C000 | FE30DFFF |
| ee_sys_ctrl | 8 | FE310000 | FE311FFF |

Link: MS1 memory map:

- AO_RESET_CTRL

This module is control module level reset which shown on diagram are “REG”.

And the control of watchdog is here also.

RESETn_module is low active.

RESETn_module = (~RESET_reg & RESET_LEVEL) | RESET_MASK;

If set RESET_reg = 1, will reset the module.

RESET_reg is an auto-recover register which will recover to 0 after some cycles.

If set RESET_LEVEL = 0, will keep RESETn_module as low, mean the module will keep at reset status.

If set RESET_MASK = 1, will keep RESETn_module as high, mean the module will never be reset.

Link: MS1_reset_ctrl

RESETCTRL_RESET0~1

RESET_reg, write each bit to 1 can reset related module, the reset will auto-cover to 0 by HW.

the list refer to resetctrl_reset0~1

RESETTRL_RESET0~1_LEVEL

write each bit to 0 will hold reset as 1.

the list refer to resetctrl_reset0~1

RESETCTRL_RESET0~1_MASK

write each bit to 1 will hold reset as 0.

the list refer to resetctrl_reset0~1

RESETCTRL_RESET_HOLD_CYC

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 2:0 | R/W | 1 | the cycle number of auto-recover |

RESETCTRL_WATCHDOG_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | watchdog status(RESET value) |
| 27 | R/W | 0 | Ao_reset_n_now |
| 26 | R/W | 0 | Sys_reset_n_now |
| 25 | R/W | 0 | Clk_div_en |
| 24 | R/W | 0 | Clk_en |
| 23 | R/W | 0 | Interrupt_en |
| 22 | R/W | 0 | Ao_reset_n_en |
| 21 | R/W |  | Sys_reset_n_en |
| 19 | R/W | 0 | Clk_sel; 0: cts_oscin_clk; 1: cts_sys_clk; |
| 18 | R/W | 0 | Watchdog_en |
| 17-0 | R/W | 0 | Clk_div_tcnt |

RESETCTRL_WATCHDOG_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 17 | R/W | 0 | Gpio_pulse |
| 16 | R/W | 0 | Gpio_polarity |
| 15-0 | R/W | 0 | Gpio_pulse_tcnt |

RESETCTRL_WATCHDOG_CNT

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31-16 | R | 0 | Watchdog count read |
| 15-0 | R/W | 0 | Watchdog_count set |

RESETCTRL_WATCHDOG_CLR

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | W | 0 | write any value will clear watchdog status |

RESETCTRL_WATCHDOG_IRQ_OFFSET

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 16 | R | 0 | Watchdog irq |
| 15:0 | W | 0 | Watchdog irq offset, make watchdog irq early than watchog rst |

- AO_PAD_CTRL

gpioW_0/1/2/3/4/5/8/9/gpioH_0/20 are OD 5V PAD,each OD 5V PAD include 6 register.

gpioW_6/7/10/11 are OD5Vx2_DPAUX PAD,

gpioW_12 is OD 3.3V CEC PAD.

all other PAD are GPIO PAD, each GPIO PAD include 6 register:

all pinmux function select option please refer to core pinmux_reg.xlsx.

Link: MS1_pinmux_register_spec.xlsx

AO_PADCTRL_PIN_MUX_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | gpioAO_7_sel |
| 27:24 | R/W | 0 | gpioAO_6_sel |
| 23:20 | R/W | 1 | gpioAO_5_sel |
| 19:16 | R/W | 1 | gpioAO_4_sel |
| 15:12 | R/W | 1 | gpioAO_3_sel |
| 11:8 | R/W | 1 | gpioAO_2_sel |
| 7:4 | R/W | 1 | gpioAO_1_sel |
| 3:0 | R/W | 1 | gpioAO_0_sel |

AO_PADCTRL_PIN_MUX_REG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 1 | gpioAO_15_sel |
| 27:24 | R/W | 1 | gpioAO_14_sel |
| 23:20 | R/W | 1 | gpioAO_13_sel |
| 19:16 | R/W | 1 | gpioAO_12_sel |
| 15:12 | R/W | 0 | gpioAO_11_sel |
| 11:8 | R/W | 0 | gpioAO_10_sel |
| 7:4 | R/W | 0 | gpioAO_9_sel |
| 3:0 | R/W | 0 | gpioAO_8_sel |

AO_PADCTRL_PIN_MUX_REG2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | gpioW_7_sel |
| 27:24 | R/W | 0 | gpioW_6_sel |
| 23:20 | R/W | 0 | gpioW_5_sel |
| 19:16 | R/W | 0 | gpioW_4_sel |
| 15:12 | R/W | 0 | gpioW_3_sel |
| 11:8 | R/W | 0 | gpioW_2_sel |
| 7:4 | R/W | 0 | gpioW_1_sel |
| 3:0 | R/W | 0 | gpioW_0_sel |

AO_PADCTRL_PIN_MUX_REG3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 23:20 | R/W | 0 | gpioW_13_sel |
| 19:16 | R/W | 0 | gpioW_12_sel |
| 15:12 | R/W | 0 | gpioW_11_sel |
| 11:8 | R/W | 0 | gpioW_10_sel |
| 7:4 | R/W | 0 | gpioW_9_sel |
| 3:0 | R/W | 0 | gpioW_8_sel |

AO_PADCTRL_GPIO_IRQ_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | gpio_irq_en |
| 11:8 | R/W | 0 | gpio_edge_en[11:8] if enable, will generate irq by fall edge or rise edge of gpio |
| 7:4 | R/W | 0 | gpio_intr_pol[7:4] 0: rise edge; 1: fall edge; |
| 3:0 | R/W | 0 | gpio_both_edge_en[3:0] if enable, will generate irq by both fall edge and rise edge of gpio |

AO_PADCTRL_GPIO_IRQ_CTRL1

| gpio src list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | src | # | src | # | src | # | src | # | src |
|  |  |  |  | 26 | gpioW_10 | 17 | gpioW_1 | 8 | gpioAO_8 |
|  |  |  |  | 25 | gpioW_9 | 16 | gpioW_0 | 7 | gpioAO_7 |
|  |  |  |  | 24 | gpioW_8 | 15 | gpioAO_15 | 6 | gpioAO_6 |
|  |  |  |  | 23 | gpioW_7 | 14 | gpioAO_14 | 5 | gpioAO_5 |
|  |  | 31 | 1’b0 | 22 | gpioW_6 | 13 | gpioAO_13 | 4 | gpioAO_4 |
|  |  | 30 | 1’b0 | 21 | gpioW_5 | 12 | gpioAO_12 | 3 | gpioAO_3 |
|  |  | 29 | gpioW_13 | 20 | gpioW_4 | 11 | gpioAO_11 | 2 | gpioAO_2 |
|  |  | 28 | gpioW_12 | 19 | gpioW_3 | 10 | gpioAO_10 | 1 | gpioAO_1 |
|  |  | 27 | gpioW_11 | 18 | gpioW_2 | 9 | gpioAO_9 | 0 | gpioAO_0 |

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 24 | R/W | 0 | gpio_irq1 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 23:21 | R/W | 0 | gpio_irq1 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 20:16 | R/W | 0 | gpio_irq1; source select; from gpio src list |
| 8 | R/W | 0 | gpio_irq0 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 7:5 | R/W | 0 | gpio_irq0 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 4:0 | R/W | 0 | gpio_irq0; source select; from gpio src list |

AO_PADCTRL_GPIO_IRQ_CTRL2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 24 | R/W | 0 | gpio_irq3 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 23:21 | R/W | 0 | gpio_irq3 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 20:16 | R/W | 0 | gpio_irq3; source select; from gpio src list |
| 8 | R/W | 0 | gpio_irq2 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 7:5 | R/W | 0 | gpio_irq2 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 4:0 | R/W | 0 | gpio_irq2; source select; from gpio src list |

AO_PADCTRL_GPIO_MSR_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 5 | R/W | 0 | gpio_msr enable |
| 4:0 | R/W | 0 | source select; from gpio src list |

AO_PADCTRL_GPIOAO_I

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R |  |  |

AO_PADCTRL_GPIOAO_O

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0xFFFF |  |

AO_PADCTRL_GPIOAO_OEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0xFFFF |  |

AO_PADCTRL_GPIOAO_IEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0x0 | 1：input enable；0：input disable |

AO_PADCTRL_GPIOAO_PULL_UP

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0x242C | 1:pull up, 0:hiz |

AO_PADCTRL_GPIOAO_PULL_DOWN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0xDBD3 | 1:pull down, 0:hiz |

AO_PADCTRL_GPIOAO_DS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0x0 | 0：ds is 0;1:ds is 1 |

AO_PADCTRL_GPIOW_I

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 12:0 | R |  |  |

AO_PADCTRL_GPIOW_O

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 12:0 | R/W | 0x1fff |  |

AO_PADCTRL_GPIOW_OEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 12:0 | R/W | 0x1fff |  |

AO_PADCTRL_GPIOW_IEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 12:0 | R/W | 0x0 | 1:input enable,0:input disable |

AO_PADCTRL_GPIOW_PULL_UP

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 12:0 | R/W | 0x0 |  |

AO_PADCTRL_GPIOW_PULL_DOWN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 12:0 | R/W | 0x0 |  |

AO_PADCTRL_GPIOW_DS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 13:0 | R/W | 0x0 | 0：ds is 0;1:ds is 1 |

AO_PADCTRL_TESTN_I

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R |  |  |

AO_PADCTRL_TESTN_O

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 0 |  |

AO_PADCTRL_TESTN_OEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 1 |  |

AO_PADCTRL_TESTN_IEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R | 0 |  |

AO_PADCTRL_TESTN_PULL_UP

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 1 |  |

AO_PADCTRL_TESTN_PULL_DOWN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 0 |  |

AO_PADCTRL_TESTN_DS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 0 |  |

AO_PADCTRL_UART_DISABLE_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 8 | R/W | 0 | uart_A_rx disable |
| 0 | R/W | 0 | uart_A_tx disable |

AO_PADCTRL_ANALOG_I

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 2 | R |  |  |
| 1 | R |  |  |
| 0 | R |  |  |

AO_PADCTRL_ANALOG_EN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 2 | WR | 0 |  |
| 1 | WR | 0 |  |
| 0 | WR | 0 |  |

AO_PADCTRL_GPIOW_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:30 | R/W | 0x2 | gpioW_6_7 aux_rxhys_sel |
| 29:28 | R/W | 0 | gpioW_6_7 aux_rx_mode |
| 27:25 | R/W | 0x4 | gpioW_6_7 aux_tx_trim |
| 24:22 | R/W | 0x4 | gpioW_6_7 aux_vref_sel |
| 21:20 | R/W | 0 | gpioW_6_7 aux_vcm_mode |
| 19 | R/W | 0 | gpioW_6_7 aux_en_rxswap |
| 18 | R/W | 0 | gpioW_6_7 aux_en_txswap |
| 17 | R/W | 0 | Release; |
| 16 | R/W | 0 | gpioW_6_7 Aux_en |
| 15:10 | R/W | 0x3F | Release; |
| 9 | R/W | 1 | gpioW_9 od_mode,1:set pad_od5v,0:pad_gpio33 |
| 8 | R/W | 1 | gpioW_8 od_mode,1:set pad_od5v,0:pad_gpio33 |
| 7:6 | R/W | 0x3 | Release; |
| 5 | R/W | 1 | gpioW_5 od_mode,1:set pad_od5v,0:pad_gpio33 |
| 4 | R/W | 1 | gpioW_4 od_mode,1:set pad_od5v,0:pad_gpio33 |
| 3 | R/W | 1 | gpioW_3 od_mode,1:set pad_od5v,0:pad_gpio33 |
| 2 | R/W | 1 | gpioW_2 od_mode,1:set pad_od5v,0:pad_gpio33 |
| 1 | R/W | 1 | gpioW_1 od_mode,1:set pad_od5v,0:pad_gpio33 |
| 0 | R/W | 1 | gpioW_0 od_mode,1:set pad_od5v,0:pad_gpio33 |

AO_PADCTRL_GPIOW_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 23:20 | R/W | 0 | gpioW_10_11 reg_dum |
| 19:18 | R/W | 0x2 | gpioW_10_11 aux_rxhys_sel |
| 17:16 | R/W | 0 | gpioW_10_11 aux_rx_mode |
| 15:13 | R/W | 0x4 | gpioW_10_11 aux_tx_trim |
| 12:10 | R/W | 0x4 | gpioW_10_11 aux_vref_sel |
| 9:8 | R/W | 0 | gpioW_10_11 aux_vcm_mode |
| 7 | R/W | 0 | gpioW_10_11 aux_en_rxswap |
| 6 | R/W | 0 | gpioW_10_11 aux_en_txswap |
| 5 | R/W | 0 | Release; |
| 4 | R/W | 0 | gpioW_10_11 Aux_en |
| 3:0 | R/W | 0 | gpioW_6_7 reg_dum |

- EE_PAD_CTRL

PAD are GPIO PAD, each GPIO PAD include 6 register:

all pinmux function select option please refer to core pinmux_reg.xlsx.

Link: MS1_pinmux_register_spec.xlsx

EE_PADCTRL_PIN_MUX_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | gpioH_7_sel |
| 27:24 | R/W | 0 | gpioH_6_sel |
| 23:20 | R/W | 0 | gpioH_5_sel |
| 19:16 | R/W | 0 | gpioH_4_sel |
| 15:12 | R/W | 0 | gpioH_3_sel |
| 11:8 | R/W | 0 | gpioH_2_sel |
| 7:4 | R/W | 0 | gpioH_1_sel |
| 3:0 | R/W | 0 | gpioH_0_sel |

EE_PADCTRL_PIN_MUX_REG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | gpioH_15_sel |
| 27:24 | R/W | 0 | gpioH_14_sel |
| 23:20 | R/W | 0 | gpioH_13_sel |
| 19:16 | R/W | 0 | gpioH_12_sel |
| 15:12 | R/W | 0 | gpioH_11_sel |
| 11:8 | R/W | 0 | gpioH_10_sel |
| 7:4 | R/W | 0 | gpioH_9_sel |
| 3:0 | R/W | 0 | gpioH_8_sel |

EE_PADCTRL_PIN_MUX_REG2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | Release; |
| 27:24 | R/W | 0 | gpioH_22_sel |
| 23:20 | R/W | 0 | gpioH_21_sel |
| 19:16 | R/W | 0 | gpioH_20_sel |
| 15:12 | R/W | 0 | gpioH_19_sel |
| 11:8 | R/W | 0 | gpioH_18_sel |
| 7:4 | R/W | 0 | gpioH_17_sel |
| 3:0 | R/W | 0 | gpioH_16_sel |

EE_PADCTRL_GPIO_IRQ_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | gpio_irq_en |
| 11:8 | R/W | 0 | gpio_edge_en[11:8] if enable, will generate irq by fall edge or rise edge of gpio |
| 7:4 | R/W | 0 | gpio_intr_pol[7:4] 0: rise edge; 1: fall edge; |
| 3:0 | R/W | 0 | gpio_both_edge_en[3:0] if enable, will generate irq by both fall edge and rise edge of gpio |

EE_PADCTRL_GPIO_IRQ_CTRL1

| gpio src list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | src | # | src | # | src | # | src | # | src |
|  |  |  |  | 26 |  | 17 | gpioH_17 | 8 | gpioH_8 |
|  |  |  |  | 25 |  | 16 | gpioH_16 | 7 | gpioH_7 |
|  |  |  |  | 24 |  | 15 | gpioH_15 | 6 | gpioH_6 |
|  |  |  |  | 23 |  | 14 | gpioH_14 | 5 | gpioH_5 |
|  |  | 31 |  | 22 | gpioH_22 | 13 | gpioH_13 | 4 | gpioH_4 |
|  |  | 30 |  | 21 | gpioH_21 | 12 | gpioH_12 | 3 | gpioH_3 |
|  |  | 29 |  | 20 | gpioH_20 | 11 | gpioH_11 | 2 | gpioH_2 |
|  |  | 28 |  | 19 | gpioH_19 | 10 | gpioH_10 | 1 | gpioH_1 |
|  |  | 27 |  | 18 | gpioH_18 | 9 | gpioH_9 | 0 | gpioH_0 |

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 24 | R/W | 0 | gpio_irq1 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 23:21 | R/W | 0 | gpio_irq1 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 20:16 | R/W | 0 | gpio_irq1; source select; from gpio src list |
| 8 | R/W | 0 | gpio_irq0 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 7:5 | R/W | 0 | gpio_irq0 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 4:0 | R/W | 0 | gpio_irq0; source select; from gpio src list |

EE_PADCTRL_GPIO_IRQ_CTRL2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 24 | R/W | 0 | gpio_irq3 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 23:21 | R/W | 0 | gpio_irq3 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 20:16 | R/W | 0 | gpio_irq3; source select; from gpio src list |
| 8 | R/W | 0 | gpio_irq2 in filter clk select: 0:8M(timebase defaule freq,can set other div freq); 1:cts_sys_clk; |
| 7:5 | R/W | 0 | gpio_irq2 in filter cycle count; filter sel 0:this valuesets the filter selection for gpio interrupt 0. A value of 0 = no filtering. A value of 7 corresponds to 7x3x(cycle time) of filtering. |
| 4:0 | R/W | 0 | gpio_irq2; source select; from gpio src list |

EE_PADCTRL_GPIO_MSR_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 5 | R/W | 0 | gpio_msr enable |
| 4:0 | R/W | 0 | source select; from gpio src list |

EE_PADCTRL_GPIOH_I

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 22:0 | R |  |  |

EE_PADCTRL_GPIOH_O

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 22:0 | R/W | 0x7F_FFFF |  |

EE_PADCTRL_GPIOH_OEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 22:0 | R/W | 0x7F_FFFF |  |

EE_PADCTRL_GPIOH_IEN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 22:0 | R/W | 0x0 | 1：input enable；0：input disable |

EE_PADCTRL_GPIOH_PULL_UP

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 22:0 | R/W | 0x70_0100 | 1:pull up, 0:hiz |

EE_PADCTRL_GPIOH_PULL_DOWN

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 22:0 | R/W | 0xFEFF | 1:pull down, 0:hiz |

EE_PADCTRL_GPIOH_DS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 22:0 | R/W | 0x0 | 0：ds is 0;1:ds is 1 |

EE_PADCTRL_UART_DISABLE_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 8 | R/W | 0 | uart_B_rx disable |
| 0 | R/W | 0 | uart_B_tx disable |

EE_PADCTRL_DEBUG_SEL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 3:0 | R/W | 0x0 | debug out sel: 0: sel mod_usb2_drd_out[1:0]; 1: sel mod_vpuhdmi_debug_bus[2:0]; 2: sel ao_test_mux[15:0]; 3: sel mod_acodec_debug_bus(reg_acodec_4[19:18] set1 :AU_DACL_VT ,reg_acodec_4[19:18] set2 : AU_DACR_VT ); |

- AO_CLK_CTRL

the chip include :

- External 24Mhz crystal

- internal PLLs:

FIXPLL

Two cfg mux is a special module for dynamic switch clock frequency.

It can generate two different clock sources by two muxs and divN.

Then selected by a “glitch free mux”.

AO_CLKCTRL_OSCIN_CTRL

Osc from XTAL pad will connect all PLL and some analog modules:

And it will gated by reg_oscin_ctrl;

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 21 | R/W | 1 | clock gate en; xtal -> VX1_PLL, PIXEL_PLL |
| 20 | R/W | 1 | clock gate en; xtal -> HIFI_PLL, DISP_PLL |
| 18 | R/W | 1 | clock gate en; xtal -> USB2_phy |
| 17 | R/W | 1 | clock gate en; xtal -> USB2(u2drd) |
| 0 | R/W | 1 | clock gate en; xtal -> PLL_TOP(FIXPLL) |

AO_CLKCTRL_RTC_BY_OSCIN_CTRL0

RTC clock is a low frequency clock which needn’t PLL.

it can generated by XTAL（base clk） of from PAD.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 1 | clock in gate en |
| 30 | R/W | 1 | clock out gate en |
| 29:28 | R/W | 0 | div_list_max |
| 23:12 | R/W | ‘d731 | clock div N1, if you want div8, set to 7 |
| 11:0 | R/W | ‘d732 | clock div N0, if you want div8, set to 7 |

AO_CLKCTRL_RTC_BY_OSCIN_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 24 | R/W | 1 | 1: force clock_out = clock_in |
| 23:12 | R/W | ‘d10 | clock div M1, if you want div8, set to 7 |
| 11:0 | R/W | ‘d7 | clock div M0, if you want div8, set to 7 |

AO_CLKCTRL_RTC_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | mux1 sel o |
| 30 | R | 0 | mux0b sel o |
| 29 | R | 0 | mux0a sel o |
| 1:0 | R/W | 0 | cts_rtc_clk source select: 0: oscin; 1:rtc from oscin div(N0/N1); 2:rtc from PAD; 3:off; |

AO_CLKCTRL_MBIST_ATSPEED_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 0 | 0: mbist clk will select oscine; 1: mbist clk will select cts_***_clk; |

AO_CLKCTRL_SYS_CLK_CTRL0

SYS_CLK is a system level clock, most of control bus and some data bus worked on this clock domain.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | no glitch mux sel out, if it’s not equal to sel in for long time, mean the FSM is dead because no clock in |
| 29 | R/W | 0 | pre_b clock gate en |
| 28:26 | R/W | 0 | pre_b source select: 0:cts_oscin_clk; 1:fclk_div2; 2:fclk_div3; 3:fclk_div4 4:fclk_div5 5:fclk_div7 6:1’b0 7:cts_rtc_clk |
| 25:16 | R/W | 0 | pre_b div N, if you want div8, set to 7 |
| 15 | R/W | 0 | no gilitch mux sel in 0: select pre_a; 1: select pre_b; |
| 13 | R/W | 1 | pre_a clock gate en |
| 12:10 | R/W | 0 | pre_a source select: 0:cts_oscin_clk; 1:fclk_div2; 2:fclk_div3; 3:fclk_div4 4:fclk_div5 5:fclk_div7 6:1’b0 7:cts_rtc_clk |
| 9:0 | R/W | 0 | pre_a div N, if you want div8, set to 7 |

AO_CLKCTRL_SYS_CLK_EN0_REG0 & AO_CLKCTRL_CLK81_GATE_CTRL

Each bit will enabled one user, all those bits are default enable after power up, if some bits are not defined, mean they are reserved.

Ref spec link: ms1_clk_spec

AO_CLKCTRL_AHB_CLK_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | no glitch mux sel out, if it’s not equal to sel in for long time, mean the FSM is dead because no clock in |
| 29 | R/W | 0 | pre_b clock gate en |
| 28:26 | R/W | 0 | pre_b source select: 0:cts_oscin_clk; 1:fclk_div2; 2:fclk_div3; 3:fclk_div4 4:fclk_div5 5:fclk_div7 6:fclk_div2p5 7:cts_rtc_clk |
| 25:16 | R/W | 0 | pre_b div N, if you want div8, set to 7 |
| 15 | R/W | 0 | no gilitch mux sel in 0: select pre_a; 1: select pre_b; |
| 13 | R/W | 1 | pre_a clock gate en |
| 12:10 | R/W | 0 | pre_a source select: 0:cts_oscin_clk; 1:fclk_div2; 2:fclk_div3; 3:fclk_div4 4:fclk_div5 5:fclk_div7 6:fclk_div2p5 7:cts_rtc_clk |
| 9:0 | R/W | 0 | pre_a div N, if you want div8, set to 7 |

AO_CLKCTRL_TST_CTRL0/1

there are 4 test source can connect to PINMUX.

each pin can select by 2bits of CLKCTRL_TST_CTRL0/1.

the test source list as blow:

| TST_CLK_OUT | Select | tst_src0_clk | tst_src1 | tst_src2 |
| --- | --- | --- | --- | --- |
| 25 | CTRL1[19:18] |  |  |  |
| 24 | CTRL1[17:16] |  |  |  |
| 23 | CTRL1[15:14] |  |  |  |
| 22 | CTRL1[13:12] |  |  |  |
| 21 | CTRL1[11:10] |  |  |  |
| 20 | CTRL1[9:8] |  |  |  |
| 19 | CTRL1[7:6] |  |  |  |
| 18 | CTRL1[5:4] |  |  |  |
| 17 | CTRL1[3:2] |  |  |  |
| 16 | CTRL1[1:0] |  |  |  |
| 15 | CTRL0[31:30] |  |  |  |
| 14 | CTRL0[29:28] |  |  |  |
| 13 | CTRL0[27:26] |  |  |  |
| 12 | CTRL0[25:24] | fclk_div2 |  |  |
| 11 | CTRL0[23:22] | fclk_div2p5 |  |  |
| 10 | CTRL0[21:20] | fclk_div3 |  |  |
| 9 | CTRL0[19:18] | fclk_div4 |  |  |
| 8 | CTRL0[17:16] | fclk_div5 |  |  |
| 7 | CTRL0[15:14] | fclk_div7 |  |  |
| 6 | CTRL0[13:12] |  |  |  |
| 5 | CTRL0[11:10] |  |  |  |
| 4 | CTRL0[9:8] |  |  |  |
| 3 | CTRL0[7:6] |  |  |  |
| 2 | CTRL0[5:4] |  |  |  |
| 1 | CTRL0[3:2] |  |  |  |
| 0 | CTRL0[1:0] |  |  |  |

AO_CLKCTRL_CECB_CLK_CTRL0

CEC_CLK always run at 32768Hz, it will generated by XTAL, but for more accuracy, it can div by two counter.

please refer to RTC_CLK for N1/N2 usage.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | Gclk_in:0:disable clk in;1:enable clk in; |
| 30 | R/W | 0 | Gclk_out: 0:disable clk out; 1: enable clk out; |
| 29-28 | R/W | 0 | Div_list_max: 0:only use clk_div0; 1:use clk_div0/clk_div1 in turn; |
| 27-24 | R/W | 0 | Reserved |
| 23-12 | R/W | 0 | Div_tcnt1: freq_clk_div1 = freq_clk_in/div_tcnt1; |
| 11-0 | R/W | 0 | Div_tcnt0: freq_clk_div0 = freq_clk_in/div_tcnt0; |

AO_CLKCTRL_CECB_CLK_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | clk_sel for cts_cec_clk 0: cecb_clk_div 1: from cts_rtc_clk |
| 30-25 | R | 0 | Reserved |
| 24 | R/W | 0 | Divide_by_1: 1: clk_out = clk_in; 0: clk_out = clk_div0 or 1; |
| 23-12 | R/W | 0 | Cycle_tcnt1 After Cycle_tcnt1 times clk_div1, clk_out will equal clk_div0; Need set div_list_max = 1; |
| 11-0 | R/W | 0 | Cycle_tcnt0 After Cycle_tcnt0 times clk_div, clk_out will equal clk_div1; Need set div_list_max = 1; |

AO_CLKCTRL_CLK12_24_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19~13 | R/ | 0 | RESERVED |
| 12 | R/W | 0 | crt_clk25_en; for CLK25M(refer to pinmux define) set 1 :CLK25M = 2GHz/ clk25_div; set 0 : CLK25M = 0; |
| 11 | R/W | 0 | crt_clk24_en; for CLK12M_24M(refer to pinmux define) set 1 :CLK12M_24M = 24M; set 0 : CLK12M_24M = 0; |
| 10 | R/W | 0 | clk24_div2_en; for CLK12M_24M(refer to pinmux define) need set crt_clk24_en = 1 first; set 1 :CLK12M_24M = 12M; set 0 : CLK12M_24M = 24M; |
| 7:0 | R/W | 0 | clk25_div; for CLK25M(refer to pinmux define) if need 8, set as 7; if set 0, will div2; etc: set 79, CLK25M = 2000MHz/80=25MHz; |

AO_CLKCTRL_PWM_CLK_(AD_)**_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 26:25 | R/W | 0 | cts_pwm_j/h/f/d/b_clk clk_sel: 0: osin 1: vid_pll 2: fclk_div4 3: fclk_fiv3 |
| 24 | R/W | 0 | cts_pwm_j/h/f/d/b_clk clk_en |
| 23:16 | R/W | 0 | cts_pwm_j/h/f/d/b_clk div N, if you want div8, set to 7 |
| 10:9 | R/W | 0 | cts_pwm_i/g/e/c/a_clk clk_sel: 0: osin 1: vid_pll 2: fclk_div4 3: fclk_fiv3 |
| 8 | R/W | 0 | cts_pwm_i/g/e/c/a_clk clk_en |
| 7:0 | R/W | 0 | cts_pwm_i/g/e/c/a_clk div N, if you want div8, set to 7 |

AO_CLKCTRL_GEN_CLK_CTRL

this register can control a special “GEN_CLK” which is designed for debug/monitor some internal clock quality, especially all PLL output. “GEN_CLK” will connect to one PAD(output).

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 16:12 | R/W | 0 | gen_clk source select 0: cts_oscin_clk; 1:cts_rtc_clk; 2:1’b0 3:1’b0 4: 1’b0 5: 1’b0 6: 1’b0 7: 1’b0 8: 1’b0 9: 1’b0 10:1’b0 11:1’b0 12:cts_msr_clk 13:1’b0 14:1’b0 15:1’b0 16: 1’b0 17: 1’b0 19: fclk_div2 20: fclk_div2p5 21: fclk_div3 22: fclk_div4 23: fclk_div5 24: fclk_div7 29:1’b0 31:1’b0 |
| 11 | R/W | 0 | gen_clk gate en |
| 10:0 | R/W | 0 | gen_clk div N, if you want div8, set to 7 |

AO_CLKCTRL_TIMEBASE_CTRL0

Some module need a fixed frequency tick as reference like UART, the tick will generated by a count which has 3 clock sources. if XTAL changed frequency, need change this register.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:24 | R/W | 0 | N1, if you want div8, set to 8 |
| 23:19 | R/W | 0 | N2, if you want div8, set to 8 |
| 18 | R/W | 0 | 1: enable xtal(24M) tick |
| 17 | R/W | 1 | Crystal clock enable. |
| 16 | R/W | 0 | Soft Reset |
| 15:14 | R/W | 0 | 1MS_TICK: 0 = use (100US_TICK/10), 1 = hold to 1, 2 = use counter1, 3 = use counter 2 |
| 13:12 | R/W | 0 | 100US_TICK: 0 = use (10US_TICK/10), 1 = hold to 1, 2 = use counter1, 3 = use counter 2 |
| 11:10 | R/W | 0 | 10US_TICK: 0 = use (1US_TICK/10), 1 = hold to 1, 2 = use counter1, 3 = use counter 2 |
| 9:8 | R/W | 0 | 1US_TICK: 0 = use (Crystal/XTAL_DIV), 1 = hold to 1, 2 = use counter1, 3 = use counter 2 |
| 7:6 | R/W | 0 | XTAL_DIV3_TICK: 0 = use Crystal/3, 1 = hold to 1, 2 = use counter1, 3 = use counter 2 |
| 5:0 | R/W | 24 | XTAL_DIV, if you want div24000, set to 24 |

AO_CLKCTRL_TIMEBASE_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 2 | R/W | 1 | 1: sync xtal_div3 by fall edge of xtal; |
| 1 | R/W | 1 | 1: sync xtal_div2 by fall edge of xtal; |
| 0 | R/W | 0 | 1: enable xtal_div2(12M) tick |

AO_CLKCTRL_HRX_CLK_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 26:25 | R/W | 0 | cts_hdmirx_2m_clkl: 0:oscin 1:fclk_div4 2:fclk_div3 3fclk_div5 |
| 24 | R/W | 0 | cts_hdmirx_2m_Clk_en |
| 22:16 | R/W | 0 | cts_hdmirx_2m_Clk div N, if you want div8, set to 7 |
| 10:9 | R/W | 0 | cts_hdmirx_5m_clkl: 0:hdmirx_aud_pll_clk |
| 8 | R/W | 0 | cts_hdmirx_5m_Clk_en |
| 6:0 | R/W | 0 | cts_hdmirx_5m_Clk div N, if you want div8, set to 7 |

AO_CLKCTRL_HRX_CLK_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 10:9 | R/W | 0 | cts_hdmirx_meter_clk sel 0:oscin 1:fclk_div4 2:fclk_div3 3:fclk_div5 |
| 8 | R/W | 0 | cts_hdmirx_meter_Clk_en |
| 6:0 | R/W | 0 | cts_hdmirx_meter_Clk div N, if you want div8, set to 7 |

AO_FIX_PLL_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |

AO_FIX_PLL_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |

AO_FIX_PLL_CTRL2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |

AO_FIX_PLL_STS

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |

- EE_CLK_CTRL

EE_CLKCTRL_TST_CTRL0/1

there are 4 test source can connect to PINMUX.

each pin can select by 2bits of CLKCTRL_TST_CTRL0/1.

the test source list as blow:

| TST_CLK_OUT | Select | tst_src0_clk | tst_src1 | tst_src2 |
| --- | --- | --- | --- | --- |
| 25 | CTRL1[19:18] | 1'b0 | 1'b0 | 1’b0 |
| 24 | CTRL1[17:16] | 1'b0 | 1'b0 | 1’b0 |
| 23 | CTRL1[15:14] | 1'b0 | 1'b0 | 1’b0 |
| 22 | CTRL1[13:12] | 1'b0 | 1'b0 | 1’b0 |
| 21 | CTRL1[11:10] | 1'b0 | 1'b0 | 1’b0 |
| 20 | CTRL1[9:8] | 1'b0 | 1'b0 | 1’b0 |
| 19 | CTRL1[7:6] | 1'b0 | 1'b0 | 1’b0 |
| 18 | CTRL1[5:4] | 1'b0 | 1'b0 | 1’b0 |
| 17 | CTRL1[3:2] | 1'b0 | 1'b0 | 1’b0 |
| 16 | CTRL1[1:0] | 1'b0 | 1'b0 | 1’b0 |
| 15 | CTRL0[31:30] | 1'b0 | 1'b0 | 1’b0 |
| 14 | CTRL0[29:28] | hrx20_pll_clk_test | hrx20_pll_lock_flag | 1’b0 |
| 13 | CTRL0[27:26] | fpll_pixel_clk | fpll_lock | 1’b0 |
| 12 | CTRL0[25:24] | audio_pll_clk | audio_pll_lock | 1’b0 |
| 11 | CTRL0[23:22] | tx_pixel_pll_clk | pix_pll_sts_i[31] | 1’b0 |
| 10 | CTRL0[21:20] | tcon_clk_out0 | p2p_pll0_lock_flag | 1’b0 |
| 9 | CTRL0[19:18] | vga_pll_clk | vga_pll_lock_flag | 1’b0 |
| 8 | CTRL0[17:16] | p20_usb2_ckout | usb_pll0_lock | 1’b0 |
| 7 | CTRL0[15:14] | disp_pll_clk | disp_pll_sts_i[31] | 1’b0 |
| 6 | CTRL0[13:12] | hifi_pll_clk | hifi_pll_sts_i[31] | 1’b0 |
| 5 | CTRL0[11:10] | fclk_div2 | fixpll_lock | am_ring_osc_clk_out_vpu[1] |
| 4 | CTRL0[9:8] | fclk_div2p5 | fixpll_lock | am_ring_osc_clk_out_vpu[1] |
| 3 | CTRL0[7:6] | fclk_div3 | fixpll_lock | am_ring_osc_clk_out_video_out |
| 2 | CTRL0[5:4] | fclk_div4 | fixpll_lock | am_ring_osc_clk_out_top[2] |
| 1 | CTRL0[3:2] | fclk_div5 | fixpll_lock | am_ring_osc_clk_out_top[1] |
| 0 | CTRL0[1:0] | fclk_div7 | fixpll_lock | am_ring_osc_clk_out_top[0] |

EE_CLKCTRL_VP_OPT_CLK_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | no glitch mux sel out, if it’s not equal to sel in for long time, mean the FSM is dead because no clock in |
| 24 | R/W | 0 | pre_b clock gate en |
| 27:25 | R/W | 0 | pre_b source select: 0:cts_oscin_clk; 1:disp_pll_clk 2:fclk_div5; 3:fclk_div4 4:fclk_div3 5:ftx_pll_div_clk 6:vdin_clk 7:tx_pixel_pll_clk |
| 22:16 | R/W | 0 | pre_b div N, if you want div8, set to 7 |
| 31 | R/W | 0 | no gilitch mux sel in 0: select pre_a; 1: select pre_b; |
| 8 | R/W | 1 | pre_a clock gate en |
| 11:9 | R/W | 0 | pre_a source select: 0:cts_oscin_clk; 1:disp_pll_clk 2:fclk_div5; 3:fclk_div4 4:fclk_div3 5:fclk_div2 6:vdin_clk 7:tx_pixel_pll_clk |
| 6:0 | R/W | 0 | pre_a div N, if you want div8, set to 7 |

EE_CLKCTRL_BCON_CLK_CNTL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 9:7 | R/W | 0 | clk_sel:: 0:oscin 1:cts_sys_clk 2:fclk_div5 3:fclk_div4 4:tx_pll_div_clk 5:1’b0 6:1’b0 7:1’b0 |
| 6 | R/W | 0 | clk_en |
| 5:0 | R/W | 0 | div N, if you want div8, set to 7 |

EE_CLKCTRL_TX_DET_CLK_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 9:7 | R/W | 0 | clk_sel:: 0:oscin 1:cts_sys_clk 2:fclk_div5 3:fclk_div4 4:1’b0 5:1’b0 6:1’b0 7:1’b0 |
| 6 | R/W | 0 | clk_en |
| 5:0 | R/W | 0 | div N, if you want div8, set to 7 |

EE_CLKCTRL_APB2_CLK_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 9:7 | R/W | 0 | clk_sel:: 0:oscin 1:cts_sys_clk 2:fclk_div4 3:fclk_div3 4:fclk_div5 5:1’b0 6:1’b0 7:1’b0 |
| 6 | R/W | 0 | clk_en |
| 5:0 | R/W | 0 | div N, if you want div8, set to 7 |

EE_CLKCTRL_SPISG_CLK_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 9:7 | R/W | 0 | clk_sel:: 0:oscin 1:cts_sys_clk 2:fclk_div4 3:fclk_div3 4:fclk_div2 5:fclk_div5 6:fclk_div7 7:1’b0 |
| 6 | R/W | 0 | clk_en |
| 5:0 | R/W | 0 | div N, if you want div8, set to 7 |

EE_CLKCTRL_PWM_CLK_(EH_)**_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 26:25 | R/W | 0 | cts_pwm_j/h/f/d/b_clk clk_sel: 0: osin 1: vid_pll 2: fclk_div4 3: fclk_fiv3 |
| 24 | R/W | 0 | cts_pwm_j/h/f/d/b_clk clk_en |
| 23:16 | R/W | 0 | cts_pwm_j/h/f/d/b_clk div N, if you want div8, set to 7 |
| 10:9 | R/W | 0 | cts_pwm_i/g/e/c/a_clk clk_sel: 0: osin 1: vid_pll 2: fclk_div4 3: fclk_fiv3 |
| 8 | R/W | 0 | cts_pwm_i/g/e/c/a_clk clk_en |
| 7:0 | R/W | 0 | cts_pwm_i/g/e/c/a_clk div N, if you want div8, set to 7 |

EE_CLKCTRL_SAR_CLK_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 10:9 | R/W | 0 | clk_sel: 0: osin 1: cts_sys_clkl 2:fclk_div5 |
| 8 | R/W | 0 | clk_en |
| 7:0 | R/W | 0 | div N, if you want div8, set to 7 |

EE_CLKCTRL_TS_CLK_CTRL

this clock is generated for temp sensor.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 8 | R/W | 0 | cts_ts_clk clk_en |
| 7:0 | R/W | 18 | cts_ts_clk div N, if you want div8, set to 7 |

EE_CLKCTRL_AXI_CLK_CTRL0

AXI_CLK is a high speed clock for most of data bus.

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R | 0 | no glitch mux sel out, if it’s not equal to sel in for long time, mean the FSM is dead because no clock in |
| 29 | R/W | 0 | pre_b clock gate en |
| 28:26 | R/W | 0 | pre_b source select: 0:cts_oscin_clk; 1:fclk_div2; 2:fclk_div3; 3:fclk_div4 4:fclk_div5 5:fclk_div7 6:fclk_div2p5 |
| 25:16 | R/W | 0 | pre_b div N, if you want div8, set to 7 |
| 15 | R/W | 0 | no gilitch mux sel in 0: select pre_a; 1: select pre_b; |
| 13 | R/W | 1 | pre_a clock gate en |
| 12:10 | R/W | 0 | pre_a source select: 0:cts_oscin_clk; 1:fclk_div2; 2:fclk_div3; 3:fclk_div4 4:fclk_div5 5:fclk_div7 6:fclk_div2p5 |
| 9:0 | R/W | 0 | pre_a div N, if you want div8, set to 7 |

EE_CLKCTRL_CLK81_GATE_CTRL & EE_CLKCTRL_AHB_CLK_GATE_CTRL

Each bit will enabled one user, all those bits are default enable after power up, if some bits are not defined, mean they are reserved.

Ref spec link: ms1_clk_spec

EE_CLKCTRL_MBIST_ATSPEED_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 0 | R/W | 0 | 0: mbist clk will select oscine; 1: mbist clk will select cts_***_clk; |

EE_CLKCTRL_GEN_CLK_CTRL

this register can control a special “GEN_CLK” which is designed for debug/monitor some internal clock quality, especially all PLL output. “GEN_CLK” will connect to one PAD(output).

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 16:12 | R/W | 0 | gen_clk source select clk0 : cts_oscin_clk clk1 : cts_rtc_clk clk2 : hifi_pll_clk clk3 : disp_pll_clk clk4 : vdin_clk clk5 : tx_pll_div_clk clk6 : tx_pixel_pll_clk clk7 : tcon_clk_out0 clk8 : vga_pll_clk clk9 : p20_usb2_ckout clk10 : 1'b0 clk11 : 1'b0 clk12 : cts_msr_clk clk13 : hrx20_pll_clk_test clk14 : audio_pll_clk clk15 : fpll_pixel_clk clk16 : 1'b0 clk17 : 1'b0 clk18 : 1'b0 clk19 : fclk_div2 clk20 : fclk_div2p5 clk21 : fclk_div3 clk22 : fclk_div4 clk23 : fclk_div5 clk24 : fclk_div7 clk25 : 1'b0 clk26 : 1'b0 clk27 : 1'b0 clk28 : 1'b0 clk29 : 1'b0 clk30 : 1'b0 clk31 : 1'b0 |
| 11 | R/W | 0 | gen_clk gate en |
| 10:0 | R/W | 0 | gen_clk div N, if you want div8, set to 7 |

EE_CLKCTRL_HRX_CLK_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 26:25 | R/W | 0 | cts_hdmirx_hdcp2x_eclk clk_sel:: 0:oscin 1:fclk_div4 2:fclk_div3 3:fclk_div5 |
| 24 | R/W | 0 | cts_hdmirx_hdcp2x_eclk clk_en |
| 22:16 | R/W | 0 | Cts_hdmirx_hdcp2x_eclk div N, if you want div8, set to 7 |
| 10:9 | R/W | 0 | cts_hdmirx_cfg_clk :clk_sel 0:oscin 1:fclk_div4 2:fclk_div3 3:fclk_div5 |
| 8 | R/W | 0 | cts_hdmirx_cfg_clk：clk_en |
| 6:0 | R/W | 0 | Cts_hdmirx_cfg_clk div N, if you want div8, set to 7 |

EE_CLKCTRL_HRX_CLK_CTRL2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 26:25 | R/W | 0 | Cts_hdmirx_acr_ref_sel:: 0:oscin 1:fclk_div4 2:fclk_div3 3:fclk_div5 |
| 24 | R/W | 0 | Cts_hdmirx_acr_ref_en |
| 22:16 | R/W | 0 | Cts_hdmirx_acr_ref_clk div N, if you want div8, set to 7 |

EE_CLKCTRL_VDOUT_CLK_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 9:7 | R/W | 0 | Cts_tcon_pll_clk_sel:: 0:oscin 1:tx_pll_div_clk 2:fclk_div5 3:fclk_div4 4:fclk_div3 5:fclk_div2 6:vdin_clk 7:tx_pixel_pll_clk |
| 6 | R/W | 0 | Cts_tcon_pll_clk_en |
| 5:0 | R/W | 0 | Cts_tcon_pll_clk div N, if you want div8, set to 7 |

- AO_SYS_CTRL

SYSCTRL_METAL_REV0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x11111111 | metal rev[31:0] |

SYSCTRL_METAL_REV1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x11111111 | metal rev[63:32] |

SYSCTRL_METAL_REV2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 11:0 | R | 0x1 | metal rev[75:64] |

SYSCTRL_CHIP_ID

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x60 | the CHIP ID |

SYSCTRL_WRITEONCE_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | each bit can write to 1 only |

SYSCTRL_WRITEONCE_REG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | each bit can write to 1 only |

SYSCTRL_WRITEONCE1_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | each bit can write to 1 only |

SYSCTRL_WRITEONCE_STICKY_REG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | each bit can write to 1 only not reset by watchdog |

SYSCTRL_WRITEONCE_STICKY_REG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | each bit can write to 1 only not reset by watchdog |

SYSCTRL_TIMERA_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 7 | R/W | A:1 B:0 C:0 D:0 G:0 H:0 I:0 J:0 | timer en |
| 6 | R/W | A:1 B:0 C:0 D:0 G:0 H:0 I:0 J:0 | timer mode: 0: generate irq once; 1: generate irq periodic; |
| 1:0 | R/W | A:0 B:0 C:0 D:0 G:0 H:0 I:0 J:0 | clk_sel: 0: cts_sys_clk; 1:1us; 2:10us; 3:100us |

SYSCTRL_TIMERA

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:16 | R | 0 | ttimer threshold |
| 15:0 | R/W | 0 | write: clean timer count; set timer threshold. irq will generate when timer count equal to threshold read: timer count value |

SYSCTRL_TIMERE/F_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 7 | R/W | E:1 F:1 | timer en |
| 1:0 | R/W | E:1 F:0 | clk_sel: 0: cts_sys_clk; 1:1us; 2:10us; 3:100us |

SYSCTRL_TIMERE/F

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | write: clean timer count; read: update timer count[63:32] to TIMER_HI timer count[31:0] |

SYSCTRL_TIMERE/F_HI

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0 | timer count[63:32] |

SYSYSCTRL_ALERT_STS_STICKY

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 4 | R | 0 | efuse write error |
| 1 | R | 0 | enclave vgd1 alert |
| 0 | R | 0 | enclave vgd0 alert |

SYSCTRL_ALERT_ENABLE

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 4 | R/W | 0 | write 1 only; enable efuse write error |
| 1 | R/W | 0 | write 1 only; enable enclave vgd1 alert |
| 0 | R/W | 0 | write 1 only; enable enclave vgd0 alert |

SYSYSCTRL_DEBUG_REG0~4

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 |  |

SYSCTRL_STATUS_REG0~3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 |  |

SYSCTRL_STICKY_REG0~3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | not reset by watchdog |

SYSCTRL_CEC_STICKY_REG0~3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | not reset by watchdog |

SYSCTRL_MSG_INDEX0~1_STICKY

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | write_en |
| 15:0 | R/W | 0 | if write_en = 1: update index = write data [15:0]; if write_en = 0: update index = index + 1 |

SYSYSCTRL_BUS_CFG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 27:4 | R/W | 0xfac688 | ahb_arbiter_pp_bits |
| 3 | R/W | 0 | ahb_arbfix_en |
| 2 | R/W | 0 | ahb_arbiter_changed |
| 1:0 | R/W | 0 | ahb_arbiter_scheme |

SYSCTRL_HDMIRXPLL_TEST_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 1 | R/W | 0x0 | hdmirxpll_test_ctrl |

SYSCTRL_SCAN_PW0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x0 | pwd_recv[31:0] |

SYSCTRL_SCAN_PW1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x0 | pwd_recv[63:32] |

SYSCTRL_SCAN_PW2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x0 | pwd_recv[95:64] |

SYSCTRL_SCAN_PW3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x0 | pwd_recv[127:96] |

SYSCTRL_TEST_MUX_SEL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 27 | R/W | 0 | 0: ao_test_mux_pre[31:16] 1:ao_test_mux_pre[15:0] |
| 2:0 | R/W | 0 | 0 ao_test_mux0 1 ao_test_mux1 2 ao_test_mux2 3 otp_info0 4 otp_info1 5 otp_info2 6 aocpu_test_mux0 7 aocpu_test_mux1 |

SYSCTRL_TEST_MUX0/1/2/3/4/5

just for debug

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R |  |  |

SYSCTRL_EFUSE_INFO0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19:4 | R | 0X0 | otp_pll_para |
| 3:2 | R | 0X0 | efuse_alert |
| 1 | R | 0X0 | efuse_ft_pwd_en |
| 0 | R | 0X0 | otp_ready |

SYSCTRL_EFUSE_INFO1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0x0 | otp_lic_0 |

SYSCTRL_EFUSE_INFO2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R | 0X0 | otp_clk_freq0 |

SYSCTRL_LDO_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 1 | R/W | 0x0 | vout_sel, 0: 2.5v; 1: 1.8v |
| 0 | R/W | 0x0 | ldo_en_09, 0：ldo_out = 0v; 1: enable |

- EE_SYS_CTRL

EE_SYSCTRL_BUS_CFG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | ARQOS_vpu_top |
| 27:24 | R/W | 0 | AWQOS_vpu_top |
| 23:20 | R/W | 0 | ARQOS_ubs2 |
| 19:16 | R/W | 0 | AWQOS_ubs2 |
| 15:12 | R/W | 0 | ARQOS_spisg |
| 11:8 | R/W | 0 | AWQOS_spisg |
| 7:4 | R/W | 0 | ARQOS_video_out |
| 3:0 | R/W | 0 | AWQOS_video_out |

EE_SYSCTRL_BUS_CFG1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:28 | R/W | 0 | ARCACHE_vpu_top |
| 27:24 | R/W | 0 | AWCACHE_vpu_top |
| 23:20 | R/W | 0 | ARCACHE_ubs2 |
| 19:16 | R/W | 0 | AWCACHE_ubs2 |
| 15:12 | R/W | 0 | ARCACHE_spisg |
| 11:8 | R/W | 0 | AWCACHE_spisg |
| 7:4 | R/W | 0 | ARCACHE_video_out |
| 3:0 | R/W | 0 | AWCACHE_video_out |

EE_SYSCTRL_NIC_CFG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 3 | R/W | 0xf | nic_ee vpu clock gate |
| 2 | R/W | 0xf | nic_ee spi clock gate |
| 1 | R/W | 0xf | nic_ee axi sram_clk clock gate |
| 0 | R/W | 0xf | nic_ee aobus_clk clock gate |

EE_SYSCTRL_VGD_CTRL

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 3 | R/W | 0x0 | vgd_en |
| 2:1 | R/W | 0x0 | vgd_lvrsel |
| 0 | R/W | 0x0 | vgd_porsel |

EE_SYSCTRL_SPISG_TRIG0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | reg_trig_mux_en |
| 30:7 | R/W | 1 | reg_dly_cnt |
| 6:5 | R/W | 0 | reg_out_sel |
| 4:0 | R/W | 0 | reg_in_sel: [19]:gpio_i [18:16]:line_n_i[2:0] [15:13]:vsync_i[2:0] [12:10]:pwm_vs_i[2:0] [9:0]:pwm_i[7:0] |

EE_SYSCTRL_OSC_RING_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 11:10 | R/W | 0 | u_am_ring_osc_vpu_1 bit1:freq_sel, 0:low frequency;1:high frequency; bit0:enable |
| 9:8 | R/W | 0 | u_am_ring_osc_vpu_0 bit1:freq_sel, 0:low frequency;1:high frequency; bit0:enable |
| 7:6 | R/W | 0 | am_ring_osc_video_out bit1:freq_sel, 0:low frequency;1:high frequency; bit0:enable |
| 5:4 | R/W | 0 | am_ring_osc_top_2 bit1:freq_sel, 0:low frequency;1:high frequency; bit0:enable |
| 3:2 | R/W | 0 | am_ring_osc_top_1 bit1:freq_sel, 0:low frequency;1:high frequency; bit0:enable |
| 1:0 | R/W | 0 | am_ring_osc_top_0 bit1:freq_sel, 0:low frequency;1:high frequency; bit0:enable |

EE_SYSCTRL_OSC_RING_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31 | R/W | 0 | am_ring_osc_resetn 0: all osc_ring is reset. |

- IRQ_CTRL

irq_ctrl include:

- collect all irq source together and sync it;

- add invert or latch if needed;

- send to different destination after masked;

IIRQCTRL_IRQ_MODE0~2

| Bits | R/W | Defaults | Description |
| --- | --- | --- | --- |
| 95:0 | R/W | 0 | if set 1, will select latched irq; if set 0, will select original irq source; |

IRQCTRL_IRQ_LATCH0~2

| Bits | R/W | Defaults | Description |
| --- | --- | --- | --- |
| 95:0 | R | 0 | the latched irq value |

IRQCTRL_IRQ_LATCH_CLR0~2

| Bits | R/W | Defaults | Description |
| --- | --- | --- | --- |
| 95:0 | R/W | 0 | if set 1, will keep latched irq to low |

IRQCTRL_IRQ_MASK0~2_AOCPU

| Bits | R/W | Defaults | Description |
| --- | --- | --- | --- |
| 95:0 | R/W | all 1 | if set 0, will keep irq as low |

RQCTRL_IRQ_INV0~2

| Bits | R/W | Defaults | Description |
| --- | --- | --- | --- |
| 95:0 | R/W | 0 | if set 1, will invert irq level |

IRQCTRL_SOFT_IRQ

| Bits | R/W | Defaults | Description |
| --- | --- | --- | --- |
| 31:8 | R/W | 0x0 | rsv |
| 7:0 | R/W | 0x0 | IRQ can be set by SW |

irq source is:

Link: MS1_irq_source

- Efuse_top

Please refer to the specific description in MS1_efuse_top_spec.docx

- aocpu

aocpu is top of N205, contain configuration register.

AOCPU_CPU_VECTOR

| Bits | R/W | Defaults | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0xF0000000 | N205 boot address |

AOCPU_CPU_CTRL0

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| [5] | R/W | 0 | nmi |
| [3] | R/W | 0 | override_dm_sleep |
| [2] | R/W | 0 | dbg_stop_at_boot |
| [1] | R/W | 0 | jtag_dwbypass |
| [0] | R/W | 0 | icache_disable_init |

AOCPU_CPU_CTRL1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| [10] | R/W | 0 | gate_by_sleep |
| [9] | R/W | 0 | RXEV |
| [7:6] | R/W | 0 | n205_debug_sel |
| [1] | R/W | 0 | n205_i_dbg_stop |
| [0] | R/W | 0 | n205_pc_enable |

AOCPU_STS00_BYCPU0~7

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | cpu read & write |

AOCPU_IRQ_TYPE0/1/2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 31:0 | R/W | 0 | aocpu_irq_type 0: pulse 1:source direct from irq_ctrl |

AOCPU_TIMER_DIV

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 7:0 | R/W | 0x3 |  |

AOCPU_TIMER_DBG_DIV

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0x600 |  |

- PWR_CTRL

defined power domain as blow：

- ao_pwr work at VDD_AO, all other work at VDD_EE;

- all power control include:

PWR_OFF(wr/rd): set 1 will cut power supply;

PWR_ACK(rd): the status of power status; 1: power is off; 0: power is on;

ISO_EN(wr/rd): set 1 will isolate output;

FOCRST(wr/rd): set 1 will keep module as reset status;

MEMPD(wr/rd): set 2’b11 will power off memory; the power of memory is different path from logic;

- For module power domain, all PWR_ACK/PWR_OFF/ISO_EN/FOCRST will follow the same sequence for each bit connection, and MEMPD will connect by different sequence.

PWRCTRL_PWR_ACK0

read only, the status of each power domain.

PWRCTRL_ISO_EN0

isolation the interface of each power domain.

PWRCTRL_PWR_OFF0

power off each power domain

| bit | module | default | bit | module | default |
| --- | --- | --- | --- | --- | --- |
| 31 |  |  | 15 |  |  |
| 30 |  |  | 14 |  |  |
| 29 |  |  | 13 |  |  |
| 28 |  |  | 12 |  |  |
| 27 |  |  | 11 |  |  |
| 26 |  |  | 10 |  |  |
| 25 |  |  | 9 |  |  |
| 24 |  |  | 8 | share_ram_top | on |
| 23 |  |  | 7 | vout_edp_top | on |
| 22 |  |  | 6 |  |  |
| 21 |  |  | 5 | video_out | on |
| 20 |  |  | 4 | vpu_top(display) | on |
| 19 |  |  | 3 | dprx_link | on |
| 18 |  |  | 2 | hdmirx_top | on |
| 17 |  |  | 1 | video_in | on |
| 16 |  |  | 0 | usb2_wrapper | OFF |

PWRCTRL_FOCRST0

| bit | module | default | bit | module | default |
| --- | --- | --- | --- | --- | --- |
| 31 |  |  | 15 |  |  |
| 30 |  |  | 14 |  |  |
| 29 |  |  | 13 |  |  |
| 28 |  |  | 12 |  |  |
| 27 |  |  | 11 |  |  |
| 26 |  |  | 10 |  |  |
| 25 |  |  | 9 |  |  |
| 24 |  |  | 8 | share_ram_top | on |
| 23 |  |  | 7 | vout_edp_top | on |
| 22 |  |  | 6 |  |  |
| 21 |  |  | 5 | video_out | on |
| 20 |  |  | 4 | vpu_top(display) | on |
| 19 |  |  | 3 | dprx_link | on |
| 18 |  |  | 2 | hdmirx_top | on |
| 17 |  |  | 1 | video_in | on |
| 16 |  |  | 0 | usb2_wrapper | OFF |

PWRCTRL_MEM_PD0

| Bit(s) | R/W | Default |  | Description |
| --- | --- | --- | --- | --- |
| 1:0 | R/W | 0x0 |  | video_in_ao |

PWRCTRL_MEM_PD1

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 19:2 | R/W | 0x0 | vpu_top(display) |
| 1:0 | R/W | 0x3 | usb |

PWRCTRL_MEM_PD2

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 23:0 | R/W | 0x0 | video_out |

PWRCTRL_MEM_PD3

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 15:0 | R/W | 0x0 | share_ram |

PWRCTRL_MEM_PD4

| Bit(s) | R/W | Default | Description |
| --- | --- | --- | --- |
| 24:0 | R/W | 0x0 | mem_pd_video_in |

- ANA_CTRL

include the register of these analog modules:

HIFI_PLL

DISP_PLL

Analog_ctrl register :MS1 PLL Application Note

- edp_lvds_phy_dig_top

include the register of these analog modules:

PLL/Panel driver

Analog_ctrl register :MS1 PLL Application Note